2-door cloud access controller

TCP/IP protocol

This agreement is confidential and may not be copied or sent to any other person without permission.

If you cannot maintain the security of this document, please delete it.

Instruction: Add a card

Command=0xC1

Software to send to device:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Card number | 3 | The unique ID number of the card, see instructions later |
| Card number | 4 | Low post in front |
| Qr Code | 9 | 9 byte string, for QR codes |
| Id card | 18 | Id number, string ACSII, no this paragraph without ID function |
| Password | 4 | 4-8 bit BCD code. F for the short part |
| Door 1 authority | 2 | Total 16 bits from low to high in binary, corresponding to 16 opening times, 1 means valid and 0 means invalid Bit2-15: Whether to select 3-16 opening hours Bit1: Whether to select the second opening time Bit0: Whether to select the first opening time |
| Door 2 authority | 2 | The same as bellow |
| Valid time | 5 | valid time begins, year month day hour minute, year -2000, year is in front; |
| Valid time | 5 | Valid time end , year month day hour minute, year -2000, year is in front; |
| Name | 8 | Name |
| Package tail | 2 |  |

Device answer:

| structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Response | 1 | 0x06: Success |
| Package Tail | 2 |  |

Here is the end of the file.