2/4-door cloud access control

HTTP Protocol

This Agreement is confidential and may not be copied or sent to any other person without permission.

If you cannot maintain the security of this document, please delete it.

Compared to the cloud +C controller, there are more parameter doors in requests and records, and more instruction relays for response opening.

Does not support face, fingerprint, vein, RFID.

Do not support anti-creep, do not support multi-card door opening.

Only 2 232 inputs are configured in the web interface for the access of a certain door. Voice and LCD can only be configured for use with a certain door.

| Date | Updates |
| --- | --- |
| 2020-06-01 | Initial Release |
| 2021-06-08 | Support for 2-door cloud controllers |

Week: 1= Sunday, 2= Monday...

How it works

The controller supports four authentication modes

Online verification priority: After swiping the card, if it is online, it will be sent to the server for verification, and no record will be saved; If it is not online, the controller verifies its own data and saves records.

Offline verification priority: after swiping the card, first judge whether the controller exists the card, if it exists, continue to judge the authority, and save the record; If the card does not exist, it is sent to the server for verification.

Online verification only: After the card is swiped, it is sent to the server for verification and no records are kept.

Offline verification only: After the card is swiped, it only determines whether the card exists and does not send it to the server; But save the card record.

The offline comparison of the above 4 modes is: ordinary IC/ID card, ID card number, QR code (9 bytes).

If it is other data, it will be automatically converted to online mode.

Network communication supports TCP or Http. You can configure one on the web interface. Only one can be used at a time.

Controller pull instruction function

The controller supports the "pull instruction" mode: every time the controller sends the heartbeat, the server software needs to answer; When the server answers, it can carry other instructions to the controller, so that the controller can complete the operation, such as opening the door, etc., which is equivalent to the controller actively asking the server whether there are instructions to be executed. The server only needs to prepare the instruction list, and the controller will automatically take it, which reduces the operation of the software to actively send instructions, reduces the difficulty of development, and improves the autonomy of the software. The most useful instruction is the personnel card data.

The pull instruction process:

The server prepares a list of data, including a instruction data, and a non-zero random serial number when the next heartbeat comes, and returns it to the controller.

The controller receives the instruction data, executes the instruction, and immediately sends the next heartbeat, which contains the serial number of the received instruction.

After receiving the heartbeat, the server analyzes the returned serial number of the instruction. If the serial number is the same as the one sent, the server considers the instruction executed successfully and generates the serial number again to send the next instruction.

If an error occurs, the serial number will change to 0. If the server has no instructions, the serial number is 0.

Note: Pull instructions are structured by the TCP protocol documentation, see the TCP protocol documentation for specific instructions. This mode is not supported by batch add card instructions.

1. Steps

- Change the computer IP and controller to the same network segment, generally 192.168.0.X.

- Open the IP address of the controller in a browser and log in , admin 888888. Chrome is recommended.

- Go to the network settings screen and change the network address and other parameters as required. Change the server IP and port of the Http, as well as the url.

If the server address is a domain name, you need to select "Address is a domain name" and fill in the DNS server IP correctly.

- Enable the Http communication function.

- Set up IIS service using http folder in demo, test using browser.

- Start a network packet capture tool (such as Ethereal) and perform packet capture to observe heartbeat and swipe card data.

- Read the documentation of this development kit in detail to develop.

- Decide whether to enable AES128 encryption after the development is complete.

Minimum items need to be completed: heartbeat data receiving and answering, access control data receiving and answering.

Key points: During the development process, use the network packet capture tool to capture the communication data of the software, compare it, and check the incorrect place of their communication packet data.

Note:

All received and sent strings are case sensitive!

The reply is a standard JSON string with double quotes, the entire reply packet is no more than 760 bytes (including http headers), and can be sent in up to 2 TCP network packets, with no extra Spaces. Any multibyte text sent is 2 bytes long.

Limit Table

| Feature | Restrictions |
| --- | --- |
| Card number length | 4 byte integer |
| Password | 5-8 digits, if not enough, pressing #, can be customized up to 12 digits |
| AES encryption | 128-bit |
| Response card number | 18 bytes for LCD board display |
| Response name | 16 bytes for LCD board display |
| Response time | 20 bytes for LCD board display |
| Response events | 32 bytes for LCD board display |
| Response speech | 40 bytes for voice board broadcast |

Http protocol Overview

The server needs to complete 4 interfaces to achieve the access control function.

- Heartbeat interface, the controller submits a request to the server at an interval (default: 5 seconds). The server can obtain whether the device is online and some status information, such as open, input, temperature and humidity and so on. The server can know whether the device is normal, and the server needs to answer.

- Access control request interface; Each time someone needs to pass, swipe the card (or other ways) that is to submit to the server access request, the server according to the obtained data for authorization judgment, the judgment results returned to the controller, the controller to make the corresponding action.

- History card record request, the record generated by offline work.

- Historical alarm record request, record generated by offline work.

The priority order of the above 4 requests is: access control request > Heartbeat request > Historical alarm record > Historical card record.

Note: Both the historical card record and the alarm record use the interface of the access control request, but the type data transmitted is different.

Protocol features supported by the controller

| NO. | Items | Length (byte) | Serial port Base64 | GET | POST | GET Encryption | POST encryption |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Heartbeat | 100 |  | √ | √ | √ | √ |
| 2 | card | 4 bytes | - | √ | √ | √ | √ |
| 3 | Password | 8 characters | - | √ | √ | √ | √ |
| 4 | Qr Code | 640 | √ | URL | JSON | DATAS | DATAS |
| 5 | Id card | About 200 | - | URL | JSON | DATAS | DATAS |
| 6 | Photo ID | 1024 | - | URL+BASE64 | JSON | DATAS&Photo=Base64 | DATAS+PHOTO= binary |

Note: All BASE64 encodings are URL safe; -_ replaces +/ and has no = symbol after it.

Request structure of the heartbeat

GET Mode

GET  /data/Acs.aspx?method=GetStatus&Now=2018060217181207&Crc=65535&T1=00&

H1=00&T2=00&H2=00&NextNum=0&Key=23685&Index=11571&Serial=R12034&Status=03&Input=0000&IndexCmd=0&Ver=135&ID=R12034&MAC=0004A33CCCB2 HTTP/1.1

Host:192.168.1.230:84

User-Agent: Chrome/61.0.3163.100 Safari/537.36

Content-Type: application/octet-stream

Connection: keep-Alive

POST Mode

POST /data/Acs.aspx?method=GetStatus& HTTP/1.1

Host:192.168.1.230:84

User-Agent: Chrome/61.0.3163.100 Safari/537.36

Content-Type: application/octet-stream

Content-Length: 222

Connection: keep-Alive

{"Serial":"R12034","Status":"03","Input":"0000","Ver":"135","ID":"R12034","IP":"192.168.3.34","MAC":"0004A33CCCB2","Now":"2018060217171607","Crc":"65535","T1":"00","H1":"00","T2":"00","H2":"00","NextNum":"0",”IndexCmd”:”0”,"Key":"26728"}

The heartbeat function is a request that the controller sends to the server after a period of time (default: 5 seconds) or when the status of the controller changes. The server can know the data and input and output status of the device, and also know whether the device is online.

You can choose to disable the heartbeat function.

Request structure of access control

GET Mode

GET/data/Acs.aspx?method=SearchCardAcs&type=9&Reader=1&Door=1&DataLen=24&Index=15319&

Serial=R12034&Status=03&Input=0000&Ver=135&ID=R12034&MAC=0004A33CCCB2&Card=ODIwNDM0MjE3NDQwMDIwNA== HTTP/1.1

Host:192.168.1.230:83

User-Agent: Chrome/61.0.3163.100 Safari/537.36

Content-Type: application/octet-stream

Connection: keep-Alive

POST Mode

POST /data/Acsx.aspx?method=SearchCardAcs HTTP/1.1

Host:192.168.1.230:83

User-Agent: Chrome/61.0.3163.100 Safari/537.36

Content-Type: application/octet-stream

Content-Length: 1133

Connection: keep-Alive

{"type":"10","Reader":"1","Door":"1","DataLen":"952","Index":"1979","Serial":"R12034","Status":"03","Input":"0000","Ver":"135","ID":"R12034","IP":"192.168.3.34","MAC":"0004A33CCCB2","Card":"S4dTUzIxAAACxMgECAUHCc7QAADixCEBAAAAgmkSmsQtAB4PxgD1AKLLygBQAKIPmgBhxBUOpABnAGUPysSSADIPRABcAHHJUgCfAIQNbQCdxKwPTgCyAI4NLsSyAGUORwALAEzKtQDzAL0P4AD1xAMPDgACAcEPTcQCAa4OlADPAbTLUwATAZwPooCZR35/GgBnhP4DmMBLB3uHw6Nyk3GzzwMf9gdyB4X1UhKTHfA9K6sRj9RIOWojxn1WDg03AMnl24H/3x34MT8a/QfW6jrMKM5+E4+P7QJPV5WtaAZ/W4tryBRca45isvpHH5YLnzR+nePru1jHICzFAMsW2gQAZwkejgcAtgUgwDpGBsTCBiLA/wXFlAvTNgYAqwYeBT4IxIYMFsD//wXALcIBWA8JQcHMAHDKDkZEwBEAiRUOBMD9wP7/wAXA/TtG/xIARR7MwMI6/sA2PsE/Og8C+icDP0A1UzoKAvIsA/9LPf/NAJ7oG0xHCQCZ9SDCBEs4BADJM+JVCMQ0NQZKMELQACyE/C/A/v5DO//9+sH/SwUAJo/9/QT8EABgTRAFK/07/v1YRg0A2lX4hUrAKMAGANJh9YX+DgBjYhA7wf6I/jJWDwBiohdJOkFF/2wKAGJrJYXAPsAWAA+r9EQ5/iv+NT5iSRMCyXnwNf4u/+z+wgVQFQAIhPCE//zj//7+wf9nBH8RxAqP6/47/Tgq/I7AwnsFAMxQLTzCAceWN8FEwwBCXXXBwWkHAJablQDFwnMHAE9lgMcEYxIACZvrOv7DP//7//3//a/CdM8BVqMr/TIEksPOAVSpPTeGQgcChLNXwP//+zoEAu23Z1YHAEMVU8I+/jYEAEnSjJsExArY7cLCT80AS3RSU/wnBQDv9gE//8AGEBEAMv3/8QcQEQYD/+4FEsgMCcBAUkLFC0HFAQALRVIAayMBAA=="}

When a traffic request occurs, the request data is submitted to the server.

The requests that can occur include: swipe card, password, QR code, ID card, button.

- Access control GET request in plain text

Card, PIN

/data/Acs.aspx?method=SearchCardAcs&type=0&Reader=0&Door=1&DataLen=1024&Index=4681&Serial=R12034&Status=03&Input=0000&Ver=135&ID=R12034&MAC=0004A33CCCB2 &Card=123456

Qr Code

/data/Acs.aspx?method=SearchCardAcs&type=1&Reader=0&Door=1&DataLen=1024&Index=4681&Serial=R12034&Status=03&Input=0000&Ver=135&ID=R12034&MAC=0004A33CCCB2 &Card=Base64 encoded data

Id card (without photo)

/data/Acs.aspx?method=SearchCardAcs&type=6&Reader=0&Door=1&DataLen=1024&Index=4681&Serial=R12034&Status=03&Input=0000&Ver=135&ID=R12034&MAC=0004A33CCCB2&Card=358212257871587

Id card (with photo)

/data/Acs.aspx?method=SearchCardAcs&type=6&Reader=0&Door=1&DataLen=1024&Index=4681&Serial=R12034&Status=03&Input=0000&Ver=135&ID=R12034&MAC=0004A33CCCB2&Card=358212257871587&Photo=Base64 encoded ID card photo data

- Access control POST request in plain text

Card, PIN

{"type":"0","Reader":"1","Door":"1","DataLen":"24","Index":"8938","Serial":"R12034","Status":"03","Input":"0000","Ver":"135","ID":"R12034","IP":"192.168.3.34","MAC":"0004A33CCCB2","Card":"1258741"}

Qr Code

{"type":"1","Reader":"1","Door":"1","DataLen":"24","Index":"8938","Serial":"R12034","Status":"03","Input":"0000","Ver":"135","ID":"R12034","IP":"192.168.3.34","MAC":"0004A33CCCB2","Card":"Base64 encoded data

Id card (without photo)

{"type":"6","Reader":"1","Door":"1","DataLen":"24","Index":"8938","Serial":"R12034","Status":"03","Input":"0000","Ver":"135","ID":"R12034","IP":"192.168.3.34","MAC":"0004A33CCCB2","Card":"3582587413694"}

Id card (with photo)

{"type":"6","Reader":"1","Door":"1","DataLen":"24","Index":"8938","Serial":"R12034","Status":"03","Input":"0000","Ver":"135","ID":"R12034","IP":"192.168.3.34","MAC":"0004A33CCCB2","Card":"3582587413694",,"Photo":" Base64 encoded ID card Photo data"}

- Access control GET Encryption request

Card, password, QR code, ID (no photo)

/data/Acs.aspx? method=SearchCardAcs&DATAS= Encrypted and then base64-encoded data HTTP/1.1

Id Card(with photo)

/data/Acs.aspx? method=SearchCardAcs&DATAS= Encrypted and then base64-encoded data &Photo= base64-encoded ID card photo data

- Access control POST encryption request

Card, PIN, QR code, ID card (without photo)

POST /data/Acsx.aspx? method=SearchCardAcs HTTP/1.1

Host: 192.168.1.230:83

User-Agent: Chrome/61.0.3163.100 Safari/537.36

Content-Type: application/octet-stream

Content-Length: 1133

Connection: keep-Alive

DATAS= json data that is encrypted and then BASE64 encoded

Id card (with photo)

DATAS= Encrypted and then BASE64 encoded json data HPOTO= binary ID card photo data

- Heartbeat request

Field Description (case sensitive) :

| FIELDS | Instructions |
| --- | --- |
| Serial | Serial number of the device |
| ID | Custom identifier of the device, set on the WEB interface. |
| Key | The key value sent, the server needs the original value returned |
| Status | The hexadecimal value of a byte, converted to binary, is 8 bits, representing the on-off status of 8 objects. Bit8-13 1-5 relay output; Bit4-7 Locked status of the 1-4 door 1 lock 0 unlocked Bit0-3 1-4 door status (off if both the door sensor and relay are off); 1 On 0 off |
| Input | Input status hexadecimal value Bit11 Reset button Bit10 tamper-proof Bit9 Fire alarm input Bit8 Alarm input Bit4-7 Door sensor 1-4 Bit0-3 Button 1-4 |
| Now | Current device time: The first 14 bytes in format: yyyyMMddHHmmss The last 2 bytes are the week, the week starts on Sunday, and Sunday is 01 |
| Index | Random value |
| Ver | Firmware Detailed version |
| MAC | The MAC address of the device |
| IndexCmd | Sequence number of last heartbeat pull instruction, original value returned |
| CmdOK | Last pull instruction execution result 0 failed 1 succeeded |

Server Response

Plain text:

HTTP/1.1 200 OK Cache-Control: private Content-Type: text/html; ..Date: Tue, 05 May 2016 08:32:33 GMT Content-Length:16....DATA={ “Key”:“543210”,“Now”:“201611261517376”,” IndexCmd”:”25874”, “CmdValue”:”02002CFF010000D003”}

Cipher text:

HTTP/1.1 200 OK Cache-Control: private Content-Type: text/html; ..Date: Tue, 05 May 2016 08:32:33 GMT Content-Length:40....DATAS={0B5981B8621B564625757DEFE0C19597}

| FIELDS | Instructions |
| --- | --- |
| Key | Mandatory parameter, the received Key value |
| Now | Optional parameters. You can compare the time of the system and the receiving device. If there is a large error, add this field. The first 14 bytes are in format: yyyyMMddHHmmss |
| AcsRes | Optional parameters, 0 Reject, 1 On, 2 Alarm, 3 Off ，4 Ignore |
| ActIndex | Optional parameters, action position, relay position, 0-4 |
| Time | Optional parameter, relay operation time, that is, how many seconds the relay holds the action. |
| Reader | Optional parameters, direction selection, such as in and out of the screen, prompt card reader, etc. |
| LCD | Optional parameters, pass=6, invalid =7, standby =0, the screen displays the page,none indicates control by the controller itself. |
| LCDTime | Optional parameters, numeric characters, how long the screen remains displayed, in seconds |
| Name | Optional parameters, name, displayed on the LCD screen, less than 16 bytes |
| Note | Optional parameters, Event description, displayed on LCD screen, less than 32 bytes |
| Card | Optional parameters, card number string, displayed on LCD screen, less than 16 bytes |
| Systime | Optional parameters, time, displayed on the LCD screen, less than 20 bytes |
| Voice | Optional parameters. voice string for broadcasting. less than, and less than 40 bytes |
| OEM | Custom features |
| IndexCmd | Optional parameters, the random sequence number with the pull instruction, 0 or none indicates that no instruction is to be executed |
| CmdValue | Optional parameters. It is a hexadecimal string that complies with the TCP communication protocol |

The access controller submits this heartbeat request every 5 seconds or when the status changes (the output changes). The server needs to answer the request.

Note: encrypted string is returned, must conform to the structure of the DATAS={0B5981B8621B564625757DEFE0C19597}

hexadecimal string is the {" Key ":" 543210 ", "Now" : "201611261517376",} encrypted value.

By default, the software answer heartbeat only answers the Key value: {" Key ", "12587"}

| Default reply | Key |
| --- | --- |
| + Open output | Key AcsRes ActIndex Time Reader |
| + Plus screen display | Key AcsRes ActIndex Time Reader LCD LCDTime Name Note Card Systime |
| + Voice broadcast | Key AcsRes ActIndex Time Reader LCD LCDTime Name Note Card Systime Voice |

Access control Request

Instructions (case sensitive)

| Fields | Remarks |
| --- | --- |
| Card | Received card number, password, QR code, ID number, etc |
| type | The received data type 0 = card; 1 = port 232 interface input character string, such as QR code; 2 = password; 3 = button request; 6 = second generation certificate card data; 9 = Base64 encoded data for serial input, QR code; 101 = historical alarm records; 100 = historical card records |
| Serial | Controller serial number, used to distinguish devices. |
| ID | Custom identification (Serial by default), which can be modified in the web interface to distinguish devices. Maximum 10 char characters |
| Reader | Card reader 0-7 corresponding to card reader |
| door. | Door 0-3 corresponding to door |
| Status | Current door status, same as this field in heartbeat |
| Index | Random value |
| MAC | MAC address of the device A string of 12 hexadecimal characters, for example 0004A33CCCB2. |
| Name | Address, ID card information Unicode encoded hexadecimal string, not longer than 12 bytes |
| Bthday | Birthday |
| Sex | 1 male 0 female |
| Nation | Ethnic decimal serial number, need to look up the table, see C# source code table, 01 indicates the Han. |
| Addr | Address, ID card information Unicode encoded hexadecimal string, not longer than 35 bytes |

Server Response

Plain text:

HTTP/1.1 200 OK Cache-Control: private Content-Type: text/html; .Date: Tue, 05 May 2015 08:32:33 GMT Content-Length:210...DATA={"Card":"4238669","Systime":"2017-01-18 11:47:09","Voice":"........,........","ActIndex":"0","AcsRes":"1","Door":"1","Time":"1","Note":"..........................","Name":"................"}

Cipher text:

HTTP/1.1 200 OK Cache-Control: private Content-Type: text/html; .Date: Tue, 05 May 2015 08:32:33 GMT Content-Length:710...DATAS={2B6D5851B9D6B67ACE56FB3319BA6EE7B7D865049851587D4F56CB62D9BF605D8CD429C9874CF86119066087FC76E1D14AC37098352183F12B7E12261BCABD69F16B4B4EC679A1AEECFA7433E031B6889BBC36C45BBC12F824311AAC354E0A34B2C976834AB4377D09481461D93D06F17429FF1068C19A0898A65732CD9F60A808BAA1455FBAFF275A4413BC55AC4D49E6DE47441C8C3C633B2ECBC1EDB29979ADC1075CE3FBD3A7906D27DF8C0FF412C3F4D8A45DFC8C51BE569F4918DD55F749431AC6D4185D3999D7292260921C41B11960DF546F59C363443B3E570567F3F83302B80B8BA91A3B2675F899C5A3E9}

The individual fields are described below:

| Fields | Instructions |
| --- | --- |
| AcsRes | Processing results,1 on, 2 Alarm, 3 Off, 0 Ignore 4 All off,100 all on |
| ActIndex | Action position, relay position: 4-door controller: 0-4, 0.. 3 corresponding to 4 doors, 4: alarm 2 door controller: 0-2, 0.. 1 corresponding to 2 doors, 2: alarm |
| Time | The relay operation time, which is how many seconds the relay holds the action. |
| LCD | Optional parameter, the screen displays pages, none indicates control by the controller itself. |
| LCDTime | Optional parameter, the time the screen remains displayed 2019-07-05 |
| Name | Optional argument, name, displayed on the LCD screen, less than 16 bytes |
| Note | Optional parameter, Event description, displayed on LCD screen, less than 32 bytes |
| Card | Optional parameter, card number string, displayed on LCD screen, less than 16 bytes |
| Systime | Optional parameter, time, displayed on the LCD screen, less than 20 bytes |
| Voice | Optional parameter.voice string for broadcasting.less than,less than 40 bytes |
| OEM | Custom customer code, none ,this field is not required |

Red 3 are required, all fields are case sensitive.

Note: Encrypt the returned string, must conform to the structure DATAS={}

Add fields as needed

| Enable output | AcsRes ActIndex Time |
| --- | --- |
| + Screen display | AcsRes ActIndex Time LCD LCDTime Name Note Card Systime |
| + Voice announcer | AcsRes ActIndex Time LCD LCDTime Name Note Card Systime Voice |

Pass-parameter selection table

|  | AcsRes | ActIndex | Action relay |
| --- | --- | --- | --- |
| Not allowed on | 0 | 0 | - |
| Allow in | 1 | 0 | Turn on Lock 1 |
| Allow Out | 1 | 1 | Turn on Lock 2 |
| Allow in | 1 | 2 | Turn on Lock 1 |
| Allow Out | 1 | 3 | Turn on Lock 2 |
| Control output | 1 | 4 | Turn on the Alarm |
| Alarm | 2 | 0 | Alarm |
| Turn off output | 3 | 0 | Turn off Lock 1 |
| Turn off output | 3 | 1 | Turn off Lock 2 |
| Turn off output | 3 | 2 | Turn off Lock 3. |
| Turn off output | 3 | 3 | Turn off Lock 4 |
| Turn off output | 3 | 4 | Turn off Alarm |
| All off | 4 | 0 | Turn off Lock 1-4 |
| All on | 100 | 0 | Turn on Lock1-4 |

History Request Record

| Fields | Remarks |
| --- | --- |
| type | 100 = History of card record; |
| Serial | Controller serial number, used to distinguish devices. |
| ID | Custom identification (Serial by default), which can be modified in the web interface to distinguish devices. Maximum 10 char characters |
| Reader | Card reader 0-7, corresponding to the card reader |
| door. | Door 0-3corresponding to doors |
| Index | Random value |
| MAC | MAC address of the device A string of 12 hexadecimal characters, for example 0004A33CCCB2. |
| IndexEvent | The serial number of the card record in the history. The server needs the original value returned |
| door. | Door number in history |
| EventType | Record type in history |
| EventTime | The time of the history |
| Card | Card number in history |
| Value | Data values in history, such as ID numbers, or QR codes |
| ValueType | This card history is of that type, as shown below 0= regular card 1= Serial port 232 interface input string, such as QR code, etc 2= Password 6= Second generation card data See ID card data structure below 9=Base64 data, from the port ,coded QR code |

9 = Historical card record response

| Fields | Instructions |
| --- | --- |
| IndexEvent | The serial number of the card record in the response history, with the original value returned |

Historical alarm record

| Fields | Remarks |
| --- | --- |
| type | 101 = historical alarm records; |
| Serial | Controller serial number, used to distinguish devices. |
| ID | Custom identification (Serial by default), which can be modified in the web interface to distinguish devices. Maximum 10 char characters |
| Reader | Card reader 0-7 |
| Index | Random value |
| MAC | MAC address of the device A string of 12 hexadecimal characters, for example 0004A33CCCB2. |
| IndexAlarm | The serial number of the alarm record in the history. The server needs the original value returned |
| door. | Door number in history |
| EventType | Record type in history |
| EventTime | The time of the history |

Historical alarm record response

| Fields | Instructions |
| --- | --- |
| IndexAlarm | The serial number of the alarm record in the response history, with the original value returned |

Base64 decoding rule: Replace the - with + , replace the _ with /  after the string,completed with a multiple of 4 =, and then decode normally

public static byte[] DecodeBase64Url(string secureUrlBase64)

{

secureUrlBase64 = secureUrlBase64.Replace('-', '+').Replace('_', '/');

switch (secureUrlBase64.Length % 4) {

case 2:

secureUrlBase64 += "==";

break;

case 3:

secureUrlBase64 += "=";

break;

}

return  Convert.FromBase64String(secureUrlBase64);

}

The End!

2020-06-01