4-door cloud access controller

TCP/IP protocol

This agreement is confidential and may not be copied or sent to any other person without permission.

If you cannot maintain the security of this document, please delete it.

Compared to the cloud +C controller, there are more parameter doors in requests and records, and more  instruction relays for response opening.

Do not support face, fingerprint, vein, RFID.

Do not support anti-pass back, do not support multi-card door opening.

Only 2 232 inputs are configured in the web interface for the access of a certain door. Voice and LCD can only be configured for use with a certain door.

2020-06-01

Update log

| Dates | Updates |
| --- | --- |
| 2020-06-01 | Initializing Version |

How it works

The controller supports 4 verification modes

Online verification priority: after swiping the card, if it is online, it is sent to the server for verification, and no record is saved; If it is not online, the controller verifies its own data and saves records.

Offline verification priority: after swiping the card, first judge whether the controller exists the card, if it exists, continue to judge the authority, and save the record; If the card does not exist, it is sent to the server for verification.

Online verification only: after swiping the card, it is sent to the server for verification and no records are kept.

Offline verification only: after swiping the card, it only determines whether the card exists and does not send it to the server; But save the card record.

The card that can be compared offline in the above 4 modes refers is: ordinary IC/ID card, ID card number, QR code (9 bytes).

Network communication supports TCP or Http protocol, configure one on the web interface, and only one can be used at the same time. This file is TCP/IP protocol.

Controller pull instruction function

The controller supports the "pull instruction" mode: every time the controller sends the heartbeat, the server software needs to answer; When the server answers, it can carry other instructions to the controller, so that the controller can complete the operation, such as opening the door, etc., which is equivalent to the controller actively asking the server whether there are instructions to be executed. The server only needs to prepare the instruction list, and the controller will automatically take it, which reduces the operation of the software to actively send instructions, reduces the difficulty of development, and improves the autonomy of the software. The most useful instruction is the personnel card data.

The pull instruction process:

The server prepares the data list, including a instruction data, and a random serial number, to the controller when the next heartbeat comes.

The controller receives the instruction data, executes the instruction, and immediately sends the next heartbeat, which contains the sequence number of the received instruction.

After receiving the heartbeat, the server analyzes the returned sequence number of the instruction. If the sequence number is the same as the one sent, the server considers the instruction executed successfully and generates the sequence number again to send the next instruction.

If an error occurs, the sequence number will change to 0. If the server has no instructions, the serial number is 0.

Note: The structure of the pull instruction is defined by this protocol document, and it also applies to the pull  instruction process of the heartbeat response in the Http protocol.

1. Development Steps

- Using demo software, conduct communication operations. Have a basic understanding and perceptual understanding of the management process of access control.

- On the basis of understanding the software, browse the instructions in the communication protocol and roughly understand the working process of the instructions.

- Understand the key instructions in all communication protocol instructions.

- Understand the instruction flow. The execution of key instructions has the order of priority.

- Master the basic structure of the instruction, complete the basic function of the instruction package.

- With the wrapper function to complete the state function response and processing, testing.

- Use wrapper functions to complete the simplest instructions such as opening the door, to test.

- Complete the communication flow framework.

Key points: During the development process, use the packet capture tool to capture the communication data of the software, make comparison, and check the incorrect place of their communication packet data.

2. Communication workflow

- Log in to the controller's IP address with your browser (chrome) and change the connection mode:

Client mode: The controller connects to the server IP address and port for communication;

Server mode: the software connects the IP and port of the controller;

- After the connection, the controller will send status and record information to the software, and the software will make corresponding response.

- In order to start the development simple, reset the controller first, and then connect, the controller will only send the status information, the software will respond; If the status message of the controller does not answer, the controller will be disconnected after more than 10 seconds.

- The software sends instructions to the controller for actions, such as opening the door.

- Swipe card, receive a swipe card event, answer the event.

3. operation steps

- Change the IP address of the computer and the controller to the same network segment, usually 192.168.0.X.

- Open the IP address of the controller in a chrome browser and log in as admin 888888

- Go to the network Settings screen and change the parameters such as the network address as needed. If the controller connection software is required, you need to change the communication mode and server IP and port.

- Enable the TCPIP communication function.

- Run demo software to test.

- Read this development kit documentation in detail to develop.

- In the development, you can use network packet capture tools (such as Ethereal) to capture network data and protocols for comparative development.

Minimum items need to be completed: heartbeat data receive and answer, swipe data receive and answer, alarm record receive and answer.

The format of the password: 4-byte integer, BCD encoding, insufficient part of F. Start with the lowest part.

Week: 1= Sunday, 2= Monday...

·

 Instruction frame structure

| Structure | Code name | Description | Instructions | Length (bytes) |
| --- | --- | --- | --- | --- |
| Packet headers | STX | Start bit | 0x02 | 1 |
|  | Rand | Random numbers | Retention | 1 |
|  | instruction | instruction | Distinguish different instructions | 1 |
|  | Address | Address |  | 1 |
|  | door. | Door number |  | 1 |
|  | LengthL | Data length low | The length of the following data section is 0, indicating that there is no data section, and the controller returns the high and low bits reversed. | 1 |
|  | lengthH | Data length high |  | 1 |
| Data | Data | Data | Data contained in the instruction, some instructions do not have this section | 0.. N |
| Packet tail | CS | Check | See the check algorithm below | 1 |
|  | ETX | End | 0x03 | 1 |

Check algorithm

Check using a simple XOR way, all the data in front of the check bit byte to perform the XOR operation to get the check value.

CS = STX xor Rand xor Command xor  Address  xor  Door xor LengthL xor LengthL xor Data[0] xor Data[1] xor Data[2] xor ….

Example: 02 AA 56 00 00 00 15 00 09 08 18 0C 07 0A 00 00 00 00 00 00 00 00 00 00 00 50 00 00 A3 03

0xA3 is the check value, the xor of all values from the beginning 0x02 to the preceding 0x00 of 0xA3.

All instruction frames must meet the following requirements:

- Meet the definition of packet header and packet end.

- The correct checksum.

- The data part and the parameters of the corresponding function of the controller receiving the instruction meet the range boundary.

Return instruction for no record:

Executed successfully: Data = 06

Unsuccessful execution: Data = 15

Note:

The length of the header part is 2 bytes, and the high and low bits of these 2 bytes in the send and receive packet are interchanged and opposite.

The value of the length field in the protocol is not fixed for each instruction. For example, the data length of the instruction 0x63 May not be 7. The length is changing, parsing protocol reference length check structure and calibration, length is not absolute value. In this way, the future protocol can be compatible with the previous version, which means that the receiver needs to ignore the additional part. [This principle applies to all communication protocols in the world.] 

List of Instructions

| Serial number | Instructions | Name | Features | Instructions |
| --- | --- | --- | --- | --- |
| 1 | 0x56 | Automatically upload status (heartbeat instruction) | Offline + online | Receiving status heartbeat |
| 2 | 0x53 | Automatically upload history swipe card record | Offline | Software controls the device after swiping the card |
| 3 | 0x54 | Automatically upload history alarm records | Offline |  |
| 4 | 0x73 | Control Output | Offline + online | Software actively controls the device |
| 5 | 0x2C | Open the door | Offline + online |  |
| 6 | 0x2D | Door normally open | Offline + online |  |
| 7 | 0x2E | close the door. | Offline + online |  |
| 8 | 0x2F | Lock the door | Offline + online |  |
| 9 | 0x5A | No card reading | Offline + online |  |
| 10 | 0x07 | Time synchronization | Offline + online |  |
| 11 | 0x18 | Operation Alarm | Offline + online |  |
| 12 | 0x19 | Operating the fire alarm | Offline + online |  |
| 13 | 0x63 | Set Controller parameters | Offline + online |  |
| 14 | 0x61 | Set the door parameters | Offline + online |  |
| 15 | 0x64 | Set the emergency door opening data | Offline + online |  |
| 17 | 0x66 | Set serial 485 parameters | Offline + online |  |
| 18 | 0x67 | Set 485 card reader function | Offline + online |  |
| 19 | 0x04 | Controller reset | Offline + online |  |
| 20 | 0x05 | Controller restart | Offline + online |  |
| 21 | 0x09 | Adding Holidays | Offline |  |
| 22 | 0x0C | Delete All Holidays | Offline |  |
| 23 | 0x0D | Increased opening hours | Off-line |  |
| 24 | 0x0F | Delete all opening hours | Offline |  |
| 25 | 0x15 | Deleting a card | Offline |  |
| 26 | 0x17 | Delete all cards | Offline |  |
| 27 | 0xC1 | Add a card | Offline | Card, ID number, QR code |
| 28 | 0x89 | Add Cards in Batches | Offline |  |
| 29 | 0x35 | Delete the swipe history | Offline |  |
| 30 | 0x3F | Delete alarm record | Offline |  |
| 31 | 0xB1 | 485 Forward data | Offline + online |  |
| 32 | 0x80 | Voice Board - Broadcast speech statements | Offline + online |  |
| 33 | 0x82 | LCD screen - Displays multiple lines of text | Offline + online |  |
| 34 | 0x83 | LCD screen - single line text display | Offline + online |  |
| 35 | 0xE7/0x57 | Access control swipe card request | Online |  |

Minimum items to be completed: heartbeat data receive and answer, swipe data receive and answer.

Offline means that the instruction makes sense if there is an offline function.

Online is where the function makes sense if there is an online function.

Offline + online means that in both cases, the instruction can be used.

Instruction: Transfer controller status

instruction=0x56

Device sends to software:

| Structure | Length | Instructions |
| --- | --- | --- |
| Package head | 7 | Command=0x56 |
| - | 1 | Spare |
| Time | 6 | The clock of the controller, from low to high: year month day minute second, year need to add 2000 |
| Gate Status | 1 | Bit5 whether exit is locked; Bit4 Whether entry is locked . Bit1 The status of exit (it is off if the door status and lock output are both off); Bit0 The state of entry (if the door status and lock output are both off, it is off); |
| Maximum number of cards | 1 | The maximum number of cards in each packet when transferring cards in bulk |
| Direction control | 1 | Bit1 exit is locked Bit0 entry is locked |
|  | 1 | Spare |
| Model number | 1 | 1 Single door |
| Relay status | 1 | Bit4 alarm relay status Bit0-3 Gate 1-4 relay status |
| Output | 2 | Status of all output ports |
| Spare | 3 |  |
| Editions | 1 | The version of the controller firmware |
| OEM code | 2 | Integer values (optional function) |
| Serial number | 6 | Serial number of the controller |
| Input port | 2 | Bit11 Reset jumper Bit10 proof Bit6-9 extends input 1-4 Bit5 Fire alarm input Bit4 alarm input Bit3 Door status sensor (Out) Bit2 Door status sensor (In) Bit1 Exit button Bit0 In button |
| Signage | 10 | In the controller's web interface can be modified, default to serial number |
|  | 16 |  |
| Firmware version | 4 | Firmware Details Version |
| Number of people left | 1 | Spare function, number of remaining passes |
| Pull instruction serial number | 4 | Serial number of last pull instruction, |
| Pull instruction result | 1 | The execution result of the last pull instruction, 1 success 0 failure |
| Sensor data | 12 * 14 | Data from the Modbus sensor; Data from all 12 sensors. |
| Pack Tails | 2 |  |

The controller actively sends status. Sends status changes either every 5 seconds, or when a pull instruction is received.

instruction=0x56

Software answer:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 | Package head address 0 indicates the entry direction, 1 indicates the exit direction, used for internal variables, select the entry and exit screen |
| Customer code high | 1 | The client code of the controller, the device with this feature must answer the correct code value |
| Customer code low | 1 |  |
| Whether to open the door | 1 | The highest bit bit7 indicates whether to control the output; 0x00 does not perform any action output; 0x81 on, 0x80 not on. The relays and opening time parameters below are required. |
| Relays | 1 | The relay number of the output, starting with 0. 0.. 4 |
| Opening time | 2 | The time of the the relay output action, in seconds. |
| Screen | 1 | Screen page, screen shows that page, 0 means controller management. The highest bit bit7 indicates whether the screen display is enabled, 0 indicates the LCD screen is enabled, 1 enable the screen; Bit0-5 indicates the screen image number |
| Display hold time | 1 | Show how many seconds hold to display the default front page. A value of 0 indicates that it is displayed until an event triggers a change. |
| Card number | 18 | Used to display card number string, 0 indicates none. |
| Sound | 40 | The string of sounds used for voice announcements, where 0 indicates none. |
| Name | 16 | Name string for display, 0 for none. |
| incident | 32 | Event string for display, 0 for none. |
| Time | 20 | Time string for display, where 0 indicates none. |
| Pull instruction sequence number | 4 | Server randomly generated, 0 indicates no subsequent pull instructions. |
| Pull instruction data | 0-N | Some instruction data, in a format that conforms to the TCP instruction structure as specified in this document. |
| Packet tail | 2 |  |

Pull instruction data: begins with 02 and ends with 03, in accordance with the protocol format of this document.

The total length of the heartbeat response including the pull instruction must not exceed 1460 bytes.

Instruction: Transmit access control request data

instruction=0xE7

When the swipe card is sent, the server validation needs to be requested online, and the device sends to the software:

| Structure | Length | Instructions |
| --- | --- | --- |
| Package head | 7 | The instruction is 0xE7 |
| Serial Number | 6 | Serial number of the controller |
| Identification | 10 | The modified identification on the Web interface, which defaults to the controller serial number |
| Card reader | 1 | Data card reader sources, 0-7 |
| - | 5 | Spare |
| Data type | 1 | 0= common card 1= serial port 232 interface input string, such as two-dimensional code, etc.; 2= password 3= button; 6= second generation card data data structure see the ID card data structure; 9=Base64 data, from the port QR code that coded; |
| System time | 16 | The first 7pcs are valid, year month day hour minute second week; Week: 1= Sunday, 2= Monday... |
| Card number | 4 | 4 byte card number |
| Data | X | Data transmitted, card number, QR code, ID card data, etc., change value, not more than 2048 |
| Packet Tail | 2 |  |

Swipe card record is the controller actively upload processing, software answer. If it is a face, the data instruction will be continuously sent until the face data is sent. The server replies when the last data is received (address=0 in the packet head indicates that the transmission is complete).

The software replies:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 | The instruction is 0x57 the address in packet head equals the received card reader |
| Open or not | 1 | If it passes the verification, the lock is unlocked. 1 open, 0 do not open, 2, alarm, 3 off, 100 in and out all open |
| Relays | 1 | The relay number of the output, starting with 0. 0.. 4 |
| Opening time | 2 | The time of the the relay output action, in seconds. |
| Screen | 1 | Screen page, screen shows that page, 0 means controller management. The highest bit , bit7 indicates whether the screen selection function is enabled, 1 is enabled, 0 is managed by access controller ,Bit0-5 indicates the screen image number, and if it is 2 screens: in and out, it is automatically determined by the entry and exit card swipe position. Pass =6, invalid =7, standby =0 |
| Hold time | 1 | Show how many seconds hold to display the default front page. If it is 0, it does not switch. |
|  | 1 |  |
| Card number | 18 | Used to display card number string, 0 indicates none. |
| Sound | 40 | The string of sounds used for voice announcements, where 0 indicates none. |
| Name | 16 | Name string for display, 0 indicates none.. |
| incident | 32 | Event string for display, 0 indicates none.. |
| Time | 20 | Time string for display, 0 indicates none. |
| Packet tail | 2 |  |

Device reply

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 | The instruction is 0x57 |
| Answer | 1 | 0x06: Success |
| Packet Tail | 2 |  |

Instruction: Transfer history swipe card record

instruction=0x53

Device sends to software:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 | instruction=0x53 |
| Card number | 4 | 4 byte card number, high in the front |
| Time | 6 | from low to high:second minute hour day month year , and the year needs to be added 2000 |
| Type of event | 1 | lower 7 digits, the type of event occurred. See TEventType in the database for a list of events The high digit indicates in and out, 0 in and 1 out |
| The door | 1 | Door address |
| Whether there are still records | 1 | Whether more records exist inside the controller |
| Record the serial number | 1 | The serial number of sending record |
| Data type | 1 | The record types, as follows 0= regular card 1= Serial port 232 interface input string, such as QR code, etc 2= Password 6= Second generation card data See ID card data structure below. 9=Base64 data, from the port QR code that coded |
|  | 1 | Spare |
| Data | 10/19 | Data value, such as ID number or QR code, length 19 if there is no ID function |
| Ancillary data | 6 | Spare |
| Packet tail | 2 |  |

Swipe card record is the controller actively upload processing, software answer. When the controller is connected, the controller actively uploads the record, so it is necessary to prepare the database and other resources before connecting the controller.

The software responds:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Record the serial number | 1 | The serial number of the received record, if any error occurs, return 0, the controller will repeat sending the last record. |
| Packet Tail | 2 |  |

Instruction: Transmit history alarm records

instruction=0x54

Device send to software:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 | instruction=0x54 |
| Time | 6 | second minute hour day month year , and the year needs to be added 2000 |
| Type | 1 | Type of event. See TEventType in the database for a list of events |
| The door | 1 | Door address, 0-1 |
| Whether there are still records | 1 | Whether more records exist inside the controller |
| Record serial number | 1 | send record serial number |
| Packet Tail | 2 |  |

The alarm record is the controller's active upload processing, and the software answers. When the controller is connected, the controller actively uploads the record, so it is necessary to prepare the database and other resources before connecting the controller.

The software responds:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Record serial number | 1 | The serial number of the received record, if any error occurs, return 0, the controller will repeat sending the last record. |
| Packet Tail | 2 |  |

Instruction: Control the output

instruction=0x73

The software actively sends the control instruction:

Used for software to actively send instructions to operate the controller switch and display and broadcast.

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 | The packet head address is equal to the card reader of the corresponding door, 0-7 |
| Whether to open the door | 1 | Whether to unlock. 1 open, 0 not open, 100 open both ways |
| Gate relay | 1 | The relay number of the output, starting from 0, 0... 4 |
| Opening time | 2 | The time of the the relay output action, in seconds. |
| Screen | 1 | Screen page, screen shows that page, 0 means controller management. The highest bit bit7 indicates whether the screen image number selection function is enabled, 1 enables, 0 controls itself, and Bit0-5 indicates the screen image number |
| Hold time | 1 | Show how many seconds hold to display the default home page, if 0 does not switch |
|  | 1 |  |
| Card number | 18 | It is used to display the card number string. All 0 indicate none. |
| Sound | 40 | A string of sounds used for voice broadcasts, All 0 indicate none. |
| Name | 16 | Name string for display,All 0 indicate none. |
| Events | 32 | Event string for display, All 0 indicate none. |
| Time | 20 | Time string for display,All 0 indicate none. |
| Packet tail | 2 |  |

Device Reply

| structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 | The instruction is 0x73 |
| Answer | 1 | 0x06: Success |
| Packet Tail | 2 |  |

Instruction: Add an opening hour

instruction=0x0D

Software sends to device:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Time area code | 1 | 0-15 |
| Start time - hours | 1 | decimal |
| Start time - minutes | 1 | decimal |
| End time - hours | 1 | decimal |
| End time - minutes | 1 | decimal |
| Week | 1 | Bit0-6: Sunday-Saturday Bit7: Have holiday permissions |
| How to open the door | 1 | Bit0-6: How to open the door 0x01: Single card identification 0x02: Card plus PIN 0x03: Timed opening and closing 0x07: Timed alarm 0x08: Password 0x09: Card or password |
| Expiration date | 3 | From low to high: year month day, year minus 2000, as in 2010-2000=10 |
| Group number | 1 | The opening hours group number for the opening hours. |
| Packet Tail | 2 |  |

Device answer:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Response | 1 | 0x06: Success |
| Packet tail | 2 |  |

Instruction: Add holidays

instruction=0x09

Software sends to device:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Holiday Index | 1 | range 1-30, holiday index must start at 1 without interruption |
| Start date - year | 1 | Year minus 2000, as in 2010-2000=10 |
| Start date - Month | 1 | decimal |
| Start date - day | 1 | decimal |
| End date - year | 1 | Year minus 2000, as in 2010-2000=10 |
| End date - month | 1 | decimal |
| End date - day | 1 | decimal |
| Packet tail | 2 |  |

Device answer:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Response | 1 | 0x06: Success |
| Packet Tail | 2 |  |

Instruction: Add a card

instruction=0xC1

Software sends to device:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Card number | 3 | The unique ID number of the card, see instructions later |
| Card number | 4 | Low post in front |
| Qr Code | 9 | 9 byte string, for QR codes |
| Id card | 18 | Id number, string ACSII, no this paragraph without ID function |
| cipher | 4 | 4-8 bit BCD code. F for the short part |
| Door 1 Permission | 1 | Total 8 bits from low to high in binary, corresponding to 8 opening times, with 1 being valid and 0 being invalid Bit2-7: Whether to select 3-8 opening hours Bit1: Whether to select the second opening time Bit0: Whether to select the first opening time |
| Door 2 Permissions | 1 | Ibid. |
| Door 3 Permissions | 1 | Ibid. |
| Door 4 Permissions | 1 | Ibid. |
| Expiration date | 5 | The period of validity begins, year month day, year -2000, year in front; |
| Valid date | 5 | Expiration date, year -2000, year in front |
| Name | 8 | Name |
| Pack tail | 2 |  |

Device answer:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Response | 1 | 0x06: Success |
| Packet Tail | 2 |  |

Instruction: Add cards in batches

Transfer multiple card records at a time, each transfer is called a package, has a package serial number.

The software sends to the device:

| Structure |  | Length (byte) | Instructions |
| --- | --- | --- | --- |
| Package head |  | 7 | instruction=0x89 |
| Packet serial number | Batch Headers | 2 | Low bit in front, start at 1 |
| Number of cards |  | 1 | The number of cards included in this package data, not more than 40. |
| standby |  | 1 |  |
| Card number |  | 3 | The unique ID number of the card, not larger than the capacity of the controller, low bit in front, starting at 0. See instructions later . |
| Card number | Number 1 data | 4 | Low bit in front |
| Cipher |  | 4 | 4-8, determined by the function value inside the 0x56 state. 5. F for the deficiency. |
| Qr Code |  | 9 | Default 9 byte string |
| Id card |  | 10 | Id number, first 16 characters BCD code compressed. No ID card function without this paragraph |
| Door Permission |  | 4 | See instructions for Adding a card |
| Status |  | 1 | 1 Valid |
| Expiration date |  | 5 | The period of validity begins, year month day, year -2000, year in front; |
| Valid date |  | 5 | Expiration date, year -2000, year in front |
| Name |  | 8 | GB2312 code |
| Card # 2 | 2nd data | 50 | Ditto for the structure of the card above |
| .. | .. |  |  |
| Packet Tail |  | 2 |  |

If it is to cancel, delete, please change the card number, password, ID card, QR code to 0.

The instruction transmits multiple card numbers at a time, the package serial number starts from 1, and the device answers the corresponding package serial number. The red part is repeated as the number of cards increases many times.

Number of cards: The number of cards in each instruction, 40 by default. This value cannot be changed.

Card number:

The card number appears only once in each package, indicating the number of the first card in the package.

The cards in each bag must be in continuous displacement according to their card number, and cannot be interrupted. If the card does not exist, you can fill in 0.

The card number in each bag must be 0, the number of cards, the number of cards *2, the number of cards *3... ... ((<= number of cards) *N) etc such a sequence, i.e. the arithmetic difference is the value of the number of cards. Formula: Card number =(package number -1) * Number of cards

Device responses:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Packet serial number | 2 | High in front |
| Packet size | 1 |  |
| Packet Tail | 2 |  |

Instruction: Controller reset

instruction=0x04

Software sends to device:

| structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 | instruction=0x04, |
| Packet tail | 2 |  |

This directive takes longer than other instructions.

Device responses:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 | instruction=0x04 |
| Answer back | 1 | 0x06: Success |
| Packet Tail | 2 |  |

instruction: Controller restarts

instruction=0x05

Software sends to device:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 | 5 |
| Packet tail | 2 |  |

After executing this instruction, the controller will start restarting within 2 seconds. The startup process takes about 1 second, but reconnecting to the network may take longer, ranging from 2 seconds to 20 seconds.

Device responses:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Response | 1 | 0x06: Success |
| Packet Tail | 2 |  |

Instruction: Time synchronization

instruction=0x07

Software sends to device:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 | instruction=0x07, |
| Time = 0x07 | 7 | Order: Second,minute,hour,week,day,month ,second is in front, base 10; 7 means Saturday; Year 10 means 2010, 24 hours a day |
| Packet Tail | 2 |  |

Note: Time data is expressed in base 10, using numbers, not characters.

Device responses:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 | instruction=0x07 |
| Answer | 1 | 0x06: Success |
| Packet Tail | 2 |  |

Instruction: Set parameter discard

instruction=0x63

Software sends to device:

| Structure | Length | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Relay status | 1 | Flip or not |
| Relay status | 1 | Prohibited or not |
| Restart time | 1 | Controller restart time, when to restart each day, the value of the specific clock hour value 0-23 |
| Verification mode | 1 | Verification mode: 0= Online verification priority 1= Offline verification priority 2= Online verification only 3= Offline verification only |
| Wait for response time | 2 | Server response timeout during online verification |
|  | 1 |  |
| Button Mode | 1 | Button whether server validation is required |
|  | 1 | Serial port input minimum data length, less than this value is ignored |
|  | 1 | Whether to close the door immediately when the door sensor is closed |
| Continuous input | 1 | The number of seconds in which the same data in the same location is prohibited from entering consecutively. |
|  | 1 | When online priority, in and out directions, forced usage patterns, 1 entry direction online judgment, 2 exit direction online judgment |
|  | 2 | A time value that disallows continuous entry of registration data. Whether to prohibit continuous input of registration data within a period, 0 indicates that the registration data is not prohibited, and non 0 indicates that the registration data is prohibited |
|  | 1 |  |
|  | 1 | When the card adds the password, whether to swipe the card directly to out of the door. |
|  | 1 |  |
|  | 1 | Double card, wait for the next input time when the card adds the password |
|  | 1 | input function configuration radio 0x01 // input do the door sensor |
|  | 1 | Input sensitivity, the lower the more sensitive, refers to the button, door sensor and other switch input |
|  | 4 |  |
|  | 2 | Fire alarm time |
|  | 2 | Alarm time |
|  | 4 | Hijack the password, in the same format as the card |
|  | 1 | Spare 0 |
|  | 1 | Interlock configuration |
| Packet tail | 2 |  |

Device answer:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Response | 1 | 0x06: Success |
| Packet Tail | 2 |  |

Instruction: Set the door parameters

instruction=0x61

Software sends to device:

| Structure | Length | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Opening time | 2 | Opening time, low in front |
| Open the door timeout | 1 | Time range for timeout detection of opening the door after the door is closed |
| Alarm for opening the door too long | 1 | When the door is opened beyond the time, an alarm record is generated. |
| Alarm type | 1 | Types of alarms, those that produce alarm output; Multiple can be selected 0x01 Door alarm 0x02 Door open too long alarm 0x04 Invalid card alarm 0x08 Invalid time alarm |
| Trigger alarm time | 2 | The output alarm time when the alarm item is triggered |
|  | 12 |  |
| Opening hours control direction | 1 | Set 1, when the group is greater than 0, then the group control the direction of the incoming, otherwise control the direction of the outgoing |
| Packet Tail | 2 |  |

Device answer:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Response | 1 | 0x06: Success |
| Packet Tail | 2 |  |

Instruction: Set emergency number parameters

instruction=0x64

Software sends to device:

| Structure | Length | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Emergency Cards | 4 * 5 | 5 emergency cards, each 4-byte integer. |
| Emergency PIN number | 4 | An emergency password |
| Emergency identity card | 18 | String ending in 0, ID card number is the string can open the door |
| Emergency string | 20 | String, ending in 0, enter the front of string including the string immediately to open the door, such as QR code scanning |
| Packet Tail | 2 |  |

Device answer:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Response | 1 | 0x06: Success |
| Packet Tail | 2 |  |

Instruction: Open the door

instruction=0x2C

Software sends to device:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 | Door address Door indicates relays, and 1-3 indicates 3 relays each |
| Packet Tail | 2 |  |

Device answer:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Response | 1 | 0x06: Success |
| Packet Tail | 2 |  |

Instruction: Door long open

instruction=0x2D

Software sends to device:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 | Door address Door indicates relays, and 1-3 indicates 3 relays each |
| Packet Tail | 2 |  |

Device answer:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Response | 1 | 0x06: Success |
| Packet Tail | 2 |  |

Instruction: Close the door

instruction=0x2E

Software sends to device:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 | Door address Door indicates the relay of the switch, and 1-3 each indicates 3 relays |
| Packet Tail | 2 |  |

Device answer:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Answer back | 1 | 0x06: Success |
| Packet Tail | 2 |  |

Instruction: alarm output

instruction=0x18

Software sends to device:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Whether to turn off alarm | 1 | Non-0 turn off alarm, 0 output alarm; The alarm time is the alarm time in the controller parameter. |
| Normally open | 1 | Whether to keep the alarm output after opening. |
| Packet Tail | 2 |  |

Device answer:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Response | 1 | 0x06: Success |
| Packet Tail | 2 |  |

instruction: Fire alarm output

instruction=0x19

Software sends to device:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Whether to turn off the fire alarm | 1 | Greater than 0 turns off the fire alarm, 0 outputs the fire alarm; The alarm time is the fire alarm time in the controller parameter. |
| Whether it is normally on | 1 | Whether to maintain fire alarm output at all times after opening |
| Packet Tail | 2 |  |

Device answer:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Response | 1 | 0x06: Success |
| Packet Tail | 2 |  |

Instruction: Lock the door

instruction=0x2F

Software sends to device:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 | Door address Door indicates relays, and 1-3 indicates 3 relays each |
| Lock status | 1 | Non-0 means lock, and 0 means unlock |
| Packet Tail | 2 |  |

Device answer:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Response | 1 | 0x06: Success |
| Packet Tail | 2 |  |

Instruction: 485 Forward data

instruction=0xB1

With this instruction, send data to 485. The software sends to the device, and the device sends the data part through the 485.

Ask to enable "485's data transfer to Instruction 0xB1"

| structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 | Address=2 in packet head |
| Forwarded data | 1-512. | Data sent by the device via 485 |
| Packet Tails | 2 |  |

After receiving the data, the 485 of the device forwards the data packet to the software:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 | Address=2 in packet header |
| Data for 485 | 1-512. | Data received by port 485 |
| Packet Tails | 2 |  |

Need to select "485’s data transfer to Instruction 0xB1" in serial configuration

Instruction: Voice Board - Broadcast voice statements

instruction=0x80

Using this instruction, data is sent to the extended 485 port and transmitted to the voice module, which broadcasts the data.

The baud rate is fixed at 9600. It is necessary to open the serial port of the controller web page and configure "Output to voice board (Baud rate 9600)".

Device sends to software:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Address | 1 | The number of the voice board, default 0 |
| Length of speech | 1 | Statement length, in bytes, no greater than 242. |
| - | 1 | Spare |
| Statement of the broadcast | 1-242. | String content |
| Pack tail | 2 |  |

Device responses:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Response | 1 | 0x06: Success |
| Packet Tail | 2 |  |

Instruction: LCD screen - Display multiple lines of text

instruction=0x82

Through this instruction, the data is sent to the first extension 485 port and transmitted to the LCD display board.

The baud rate is fixed at 9600, you need to open the controller web page serial port configuration "Output to LCD screen (Baud rate 9600)".

Device sends to software:

| structure | Length | Instructions |
| --- | --- | --- |
| Package head | 7 | Package head address 0 means door screen, 1 means door screen |
| Time-lapse display home page | 1 | How long the display becomes the default interface of the home page, in seconds |
| Pages | 1 | Use to display different background pictures. 0 standby, 6 Pass, 7 fail, other pages can be customized |
| Card Number | 18 | To display the card number string, need the card number sticker of LCD screen as string |
| Name | 16 | Name string to display |
| Events | 32 | Event string to display |
| Time | 20 | Time string to display |
| Packet tail | 2 |  |

Device answer:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Answer back | 1 | 0x06: Success |
| Packet Tail | 2 |  |

Instruction: LCD screen - single line text display

instruction=0x83

With this instruction, data is sent to the extended 485 port to display a line of text on the LCD display board. By using this  instruction on the customized LCD screen, you can display any effect you want.

The baud rate is fixed at 9600, need to open the controller web page serial port configuration "Output to LCD screen (Baud rate 9600)".

Device sends to software:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 | Package head address 0 means door screen, 1 means door screen |
| page | 1 | Use to display different background pictures. 0 standby, 6 Pass, 7 fail, other pages can be customized |
| Address | 1 | The displayed position, optional values are 0x99 0x66 0x77 0x88 |
| Display time | 1 | Display the content retention time, in seconds, if the time is exceeded, the default interface is displayed |
| Display content | 32 | Become longer , the content string to be displayed. |
| Packet tail | 2 |  |

Device answer:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Response | 1 | 0x06: Success |
| Packet Tail | 2 |  |

Instruction: Delete all history card record  instruction=0x35

Instruction: Delete all history alarm record  instruction=0x3F

Instruction: Delete open time			  instruction=0x0F

Instruction: Delete all card data  instruction=0x17

Instruction: Delete all Holiday data  instruction=0x0C

Software sends to device:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Packet tail | 2 |  |

Device answer:

| Structure | Length (byte) | Instructions |
| --- | --- | --- |
| Package head | 7 |  |
| Response | 1 | 0x06: Success |
| Packet Tail | 2 |  |

Id card data format instructions

| Items | Length (bytes) | Instructions |
| --- | --- | --- |
| Whether there is a photograph | 1 | If it is 1, it means there is a photo section behind it, otherwise there is no photo data. |
| Name | 12 | Chinese characters UTF-16 |
| Gender | 1 | Code |
| Nationalities | 4 | CODE |
| Birth | 8 | Year ,month,day : YYYYMMDD |
| ADDRESS | 70 | Chinese characters and numbers UTF-16 |
| People card number | 18 | Numbers |
| Issuing authority | 30 | Chinese characters UTF-16 |
| Validity start date | 8 | Year ,month,day : YYYYMMDD |
| Validity end date | 8 | Year ,month,day : YYYYMMDD |
| PHOTOS | 1024 |  |

Text information in GB 13000 UCS-2

Here is the end of the file.