4-door cloud access control controller

Mixed version

Online working principle

After swiping the card (password, ID card, QR code) on the access control controller, it is transmitted in real time to the cloud server. The server processes the received data and responds to the controller. The controller responds accordingly, such as opening the door. It can also provide voice broadcasting and LCD screen display.

Require network communication between devices and servers, with optional TCP and HTTP protocols.

Working principle of offline

Swipe the card on the access control controller, and the controller directly determines whether it belongs to the registration information (card, password, ID card, QR code), and the controller directly determines whether to open the door. Optional TCP and HTTP protocols.

4 verification modes

Priority for online verification: After swiping the card, if it is online, it will be sent to the server for verification without saving records; If not online, the controller will verify its own data and save records.

Offline verification priority: After swiping the card, first check if the controller has the card. If it does, continue to check permissions and save records; If the card does not exist, send it to the server for verification.

Online verification only:
       After swiping the card, send it to the server for verification without saving records.

Offline verification only:
       After swiping the card, only check if the card exists and do not send it to the server; But keep a record of card swiping.

Advanced features

Support the controller to directly initiate pull command data, making the communication process simpler.

The controller pulls the instructions that need to be executed from the server through the heartbeat, so that the server only needs to prepare the instruction data and does not need to actively send instructions to the controller, reducing the complexity of communication development and operation. This feature supports both HTTP and TCP protocols.

characteristic

- Supports 8 WG card reader interfaces, supports protocols such as WG2 and WG34, and can be configured through the web.

- 1 set of 485 interfaces, 4 sets of button switch inputs, 2 sets of alarm inputs, 4 relay control outputs, and 1 relay alarm.

- Directly supports two 232 inputs, such as QR codes and ID card collectors.

- 485 supports voice module, real-time live broadcast of synthesized voice, such as "Welcome, Manager Li!", controlled by server software.

- 485 supports 485 type readers, including 485 readers with display screens.

- Supports LCD screen display, achieving on-site screen display.

- Supports TCP/IP and HTTP protocols.

- The HTTP protocol supports domain name and IP address servers, supports custom URL paths, and is suitable for cloud application development.

- HTTP supports both Post and Get methods for submitting data, with both options available. It also supports long connection methods.

- Supports AES128 encryption, custom keys, and more secure data.

- Support the controller to directly initiate pull command data, making the communication process simpler.

parameter

| source | 12VDC |
| --- | --- |
| interface | 1 RJ45, 2 232 ports, DB9 pins , and 1 485 interface |
| input | 4 buttons , 4 door magnets , 1 alarm , and 1 fire alarm |
| Access control request | 2 swipe cards, 2 (QR code, ID card) |
| output | 4 locks control 1 alarm |
| Reading Head | 8 WG interfaces |
| four hundred and eighty-five | Poris 485 Reading Head LCD Reading Head LCD Display Screen Voice Output |
| Power output | 1 USB power port (5VDC), 1 set of 5V DC power supply |
| Support Protocol | HTTP TCP |
| PCB size | 100.5mm * 210mm |
| Card capacity |  |
| Card swiping records | thirteen thousand |
| Alarm records | three thousand |

End of file!