Access controller API interface--wiegand input format

SECURITY TIP:

This protocol is a confidential document . Please do not copy or send to others without permission.

If you can not maintain doc. Security, please delete it.

Introduction

We can define controller’s wiegand input format.

- Given reader type

- Given data’s verification algorithm or not verify it.

- Can limit some cards

- Card no. Reverse output

This file is used for new CPU: Cloud+ controller, cloud 2 door, cloud 4 door, turnstile, single door, Embedded controller.

Software sents http instruction to controller and controller respond.

Instruction address is controller IP, +URL, port of fixed 80

| Function | Type | Instruction | Explanation |
| --- | --- | --- | --- |
| Get config | GET | http://192.168.0.131/HttpApi.xml?Cmd=GetWGFormat | Get all configs inside device |
| Save config | POST | http://192.168.0.131/SaveWGFormat | Save config into device |
| Reset | GET | http://192.168.0.131/HttpApi.xml?Cmd=ResetWGFormat | Reset--back to device default config |

Note: fixed 10 configs, get and save config will have all 10 together every time.

1 Get config http://192.168.0.131/HttpApi.xml?Cmd=GetWGFormat

Function：Get all configs from device

Software sends

GET  http://192.168.0.131/HttpApi.xml?Cmd=GetWGFormat HTTP/1.1

Host:192.168.1.230:84

Content-Type: application/json

Controller respond

HTTP/1.1 200 OK

User-Agent: Chrome/61.0.3163.100 Safari/537.36

Cache-Control: no-cache

Content-Type: application/json

Content-Length:1055

{"err_code":"0","message":"",

"Enable":[1,1,1,1,1,0,0,0,0,0],

"Len":[26,34,35,37,66,0,0,0,0,0],

"OddParity":[1,1,0,0,1,0,0,0,0,0],

"EvenParity":[26,34,0,0,66,0,0,0,0,0],

"CheckOdd":[1,1,0,0,1,0,0,0,0,0],

"CheckEven":[1,1,0,0,1,0,0,0,0,0],

"FacCode":[0,0,0,0,0,0,0,0,0,0],

"ByteDir":[0,0,0,0,0,0,0,0,0,0],

"IncFac":[0,0,0,0,0,0,0,0,0,0],

"OddMask":["00000000000000001FFE","0000000000000001FFFE","00000000000000000000",

"00000000000000000000","000000000001FFFFFFFE","00000000000000000000","00000000000000000000",

"00000000000000000000","00000000000000000000","00000000000000000000"],

"EvenMask":["00000000000001FFE000","000000000001FFFE0000","00000000000000000000",

"00000000000000000000","0001FFFFFFFE00000000","00000000000000000000","00000000000000000000",

"00000000000000000000","00000000000000000000","00000000000000000000"],

"CardMask":["0000000000000001FFFE","000000000001FFFFFFFE","0000000000000001FFFE",

"0000000000000001FFFE","0001FFFFFFFFFFFFFFFE","00000000000000000000","00000000000000000000",

"00000000000000000000","00000000000000000000","00000000000000000000"]}

Field explanation, each field has the same number group length of 10

| Field | Type | Explanation |
| --- | --- | --- |
| Enable | UInt8 | Each config enable or not, 1 Enable, 0 Disable |
| Len | UInt8 | Reader data length: data bit from reader like 26 or 34 |
| OddParity | UInt8 | Position of odd parity value，start from 1 |
| EvenParity | UInt8 | Position of even parity value, start from 1 |
| CheckOdd | UInt8 | Enable odd parity or not, 1 Enable, 0 disable |
| CheckEven | UInt8 | Enable even parity value, 1 Enable, 0 disable |
| FacCode | UInt8 | Show facility code or not, 0 not show facility code, or card’s facility code will be proceeded as the same as this value (only for 26/34) |
| ByteDir | UInt8 | Card no. Output sequence,0 default, 1 revers order by byte, 2 reverse order by half byte |
| IncFac | UInt8 | output card no. Incude facility code or not, default include, 1 include, 0 not contain |
| OddMask | Hex | Hexadecimal character string,Mask of all bits involved in odd parity, position of 1 involved in check |
| EvenMask | Hex | Hexadecimal character string,Mask of all bits involved in even parity, position of 1 involved in check |
| CardMask | Hex | Hexadecimal character string, card no.’s mask,position of 1 involved in check |

2 Reset config  http://192.168.0.131/HttpApi.xml?Cmd=ResetWGFormat

Function:Reset config to device default config.

Software sends

GET /HttpApi.xml?Cmd=ResetWGFormat HTTP/1.1

Content-Type: application/json

Host: 192.168.0.131

Connection: Keep-Alive

Controller answer

HTTP/1.1 200 OK

User-Agent: Chrome/61.0.3163.100 Safari/537.36

Cache-Control: no-cache

Content-Type: application/json

Content-Length:29

{"err_code":"0","message":""}

3 Store config  http://192.168.0.131/SaveWGFormat

Function:Store config parameters into device

Software sends

POST /SaveWGFormat HTTP/1.1

Content-Type: application/json

Host: 192.168.0.131

Content-Length: 1027

Expect: 100-continue

{"EvenParity":[26,34,0,0,66,0,0,0,0,0],

"ByteDir":[0,0,0,0,0,0,0,0,0,0],

"Enable":[1,1,1,1,1,0,0,0,0,0],

"EvenMask":["00000000000001FFE000","000000000001FFFE0000","00000000000000000000",

"00000000000000000000","0001FFFFFFFE00000000","00000000000000000000","00000000000000000000",

"00000000000000000000","00000000000000000000","00000000000000000000"],

"CheckOdd":[1,1,0,0,1,0,0,0,0,0],

"CheckEven":[1,1,0,0,1,0,0,0,0,0],

"IncFac":[1,1,1,1,1,0,0,0,0,0],

"OddMask":["00000000000000001FFE","0000000000000001FFFE","00000000000000000000",

"00000000000000000000","000000000001FFFFFFFE","00000000000000000000","00000000000000000000",

"00000000000000000000","00000000000000000000","00000000000000000000"],

"CardMask":["00000000000001FFFFFE","000000000001FFFFFFFE","0000000000000001FFFE",

"0000000000000001FFFE","0001FFFFFFFFFFFFFFFE","00000000000000000000","00000000000000000000",

"00000000000000000000","00000000000000000000","00000000000000000000"],

"FacCode":[0,0,0,0,0,0,0,0,0,0],

"OddParity":[1,1,0,0,1,0,0,0,0,0],

"Len":[26,34,35,37,66,0,0,0,0,0]}

Controller answers

HTTP/1.1 200 OK

User-Agent: Chrome/61.0.3163.100 Safari/537.36

Cache-Control: no-cache

Content-Type: application/json

Content-Length:29

{"err_code":"0","message":""}

See front “get instruction explanation” for field description

THE END