WebApi (HTTP) protocol

Safety Tips

This agreement is a confidential document and may not be copied or sent to other personnel without permission.

If you are unable to maintain the security of this file, please delete it.

introduce

This document describes the data format for communication between controllers and computer servers, which is based on the HTTP protocol and uses JSON structure for data packets. This article describes and defines the specific structure of the JSON.

This document is suitable for the following models (supported by domestic chips):

Single door, two door, four door, gate machine, integrated machine;

Cloud controller, cloud 2-door controller, cloud 4-door controller;

Elevator controller;

characteristic

- Supports the HTTP protocol, sends instructions to the controller through HTTP, facilitating network interface docking.

- The WebAPI interface has good scalability and compatibility.

- The protocol data uses a JSON structure, making development and integration very simple.

- The model is complete and meets various project requirements.

- Does not support HTTPS encryption, push optional support for HTTPS.

- Support batch downloading.

- Push heartbeat status information and record information through a callback interface, without the need for polling.

- Provide customized interface services to meet personalized needs.

- Support heartbeat pull command to download data, making server development simple and efficient.

# HTTP

The access control controller is the HTTP server, and the software connects to the HTTP server of the controller.

At the same time, configure a callback URL interface in the controller, so that when the controller generates a heartbeat and records, it is pushed to the software platform.

The interface call method is POST.

Default interface: http://192.168.3.254/HttpApi?ApiKey=12f587e4

Among them, 192.168.1.254 is the IP address of the controller, and each controller has a default IP address, as indicated by the label sticker; After logging in to this IP address using a browser, you can continue with various parameter configurations.

/HttpApi? ApiKey=12f587e4 is a fixed interface URL, while ApiKey=12f587e4 is used for personalized customization of parameters.

The data for a complete interface call is as follows:

POST/HttpApi? ApiKey=12f587e4 HTTP/1.1

Content Type: application/JSON

User Agent: PostmanRuntime/7.35.0

Accept:*/*

Cache Control: no cache

Host: 192.168.1.254

Connection: keep alive

Content Length: 144

{"id": 2017, "data": {"Door": 0, "Open": 1}, "version": "1.0", "taskNo": 10, "method": "OpenDoor"}

HTTP/1.1 200 OK

User Agent: Chrome/61.0.3163.100 Safari/537.36

Cache Control: no cache

Content Type: application/JSON

Content Length: 131

{"id": 2018, "taskNo": 10, "method": "OpenDoor", "version": "1.0", "code": 0, "Serial": "840254", "ID": "840254", "message": "......", "data": {}}

An example of executing an API instruction is as follows (query status):

Basic structure of communication JSON

Requestor:

{

"ID": 1,

"Version": "1.0",

"TaskNo": 10,

Method:,

"Data": {}

}

Respondent:

{

"ID": 1,

"Version": "1.0",

"Code": "200",

"Message": "",

"TaskNo": 10,

"Method": "

}

General Field Description

| field | data type | explain |
| --- | --- | --- |
| ID | UInt32 | Serial number |
| Version | String | Instruction version |
| Code | String | The result code of instruction execution, specific meaning can be found in the Error Code List |
| Message | String | Error description returned when there is an error |
| TaskNo | UInt32 | The initiator of the task number defines that the receiver has processed the original value of the task and returned it |
| Data | Json object | JSON Data Object |
| Method | String | Instruction, the function of this instruction is the same as that of the responder and sender |

Special note: All JSON field names, values, and symbols are case sensitive!

Unless otherwise specified in the document, the unit of time length is seconds.

# Data type description

| data type | explain |
| --- | --- |
| UInt8 | 8-bit unsigned integer, occupying 1 byte |
| UInt16 | 16 bit unsigned integer, occupying 2 bytes |
| UInt32 | 32-bit unsigned integer, occupying 4 bytes |
| String | character string |
| Array | Json structure array, square brackets [] |
| Json object | JSON data object, {} or [] |

# Error code table

| code | meaning | explain |
| --- | --- | --- |
| 0 | Execution successful |  |
| one | Unsupported Instructions | This model or version does not support this command |
| two | Not executed successfully |  |
| three | JSON does not exist | Missing a variable field, incorrect capitalization |
| four | Illegal JSON | JSON syntax is incorrect or not recognized. |
| five | Illegal JSON value | The value of the field is in illegal JSON format. |
| six | Value out of range | The value of the field exceeds the expected range. |
| seven | No data found in search |  |
| eight | Inconsistent number of parameters | The instruction requires multiple array variables, but the length of the array is inconsistent. |

# Instruction directory

| instructions | Instruction Definition | explain |
| --- | --- | --- |
| Status | Report controller heartbeat | Proactively send URLs to the server through push notifications |
| Event | Reporting Records | Proactively send URLs to the server through push notifications |
| GetStatus | Query the status value of the controller | Data reported with heartbeat |
| SetNetwork | Set network parameters | Used to configure callback interfaces |
| OpenDoor | Opening and closing doors |  |
| OpenDoorLong | The door is long open |  |
| LockDoor | (Unlocking) the door |  |
| SetAlarm | Alarm |  |
| SetFire | Fire alarm |  |
| SetPass | Control entry and exit |  |
| DeleteTimeZone | Initialize open hours |  |
| AddTimezone | Add opening hours |  |
| ClearCards | Clear all cards |  |
| DeleteCard | Delete Card |  |
| AddCard | Add Card |  |
| AddAllCards | Batch Add Card - Numbering Mode | recommendation |
| AddListCards | Batch Add Card - No Number Mode |  |
| SearchCard | Search Card | Search by card number, number, and password |
| UpdateCardAPB | Update card anti backflow |  |
| ClearHolidays | Clear all holidays |  |
| AddHoliday | Add holidays |  |
| TransmitTo485 | Sending data through 485 | Controller support with 485 interface |
| Speak | Voice Announcements | Controller support with 485 interface |
| UpdateTime | Synchronization time | NTP replacement can be initiated, and controller web interface configuration can be enabled |
| Restart | reboot device |  |
| Reset | Initialize device |  |
| SetDoor | Set door parameters |  |
| SetController | Set controller parameters |  |

The data structure for reporting heartbeat status and events varies depending on the device model and is divided into two categories:

The structure of the standard controller and the Mqtt protocol can be found in the Mqtt protocol.

The data structure of the cloud controller and its corresponding HTTP protocol can be found in the cloud controller HTTP protocol documentation.

Minimum docking (must be completed): Status, Event, AddCard

Docking stairs, docking in sequence

- Status

- OpenDoor

- Event

- AddCard

- ClearCards, DeleteCard, Reset

- DeleteTimeZone, AddTimezone

- AddAllCards

- UpdateTime

- AddHoliday, ClearHolidays

- SetDoor, SetController

Model Code Table

| Model code | classification | Model Description |
| --- | --- | --- |
| 0x71 | Single door | Practical TCP Single Gate |
| 0x9E |  | TCP network all-in-one machine -3rd generation |
| 0x90 |  | TCP gate controller -3rd generation |
| 0x91 |  | TCP Single Gate-3rd Generation |
| 0x72 | Two doors | Practical TCP Dual Gate |
| 0x92 |  | TCP Dual Gate Generation 3 |
| 0x94 | Four Gates | TCP Four Gates - Generation 3 |
| 0x9C | Cloud gate machine | Cloud Gate Machine Generation 3 |
| 0xB4 | 云+ | Cloud+B controller 4th generation, only working in online mode |
| 0xC4 |  | Cloud+C controller 4th generation, capable of offline and online hybrid mode |
| 0xCB | Cloud Two Gates | Cloud+2 door controller 4th generation |
| 0xCD | Cloud Four Gates | Cloud+4 door controller 4th generation |
| 0xE3 | Cloud elevator | Cloud Elevator Controller 3rd Generation |

# Controller sends heartbeat

Controller pushes to heartbeat URL

| Controller request |  |  | Software response |
| --- | --- | --- | --- |
| { "ID": 9, "Version": "1.0", "Method": "Status", "TaskNo": 10, "Data":{ "Serial": "1G0095", "ID": "1C1110", DoorStatus: [0,0], "Input": 0, "Now": "2022-12-09 12:34:48", "Firmware": "0.1.121", "BuildTime": "Dec 9 2022 12:34:08", "PCBCode": "DD10", "Model": "G-ZJ1000TCP", TimeStamp: 1670589288, "CardsInPacket": 16, "SystemOption": 57, DoorRelay: [0,0], "Alarm Relay": [0,0], "Button": [0,0], "Sensor": [0,0], "Alarm In": 0, "FireIn": 0, "TamperIn": 0, Reset In: 0 } } |  |  | { "Method": "Status", "Code": "0", "TaskNo": 10, "Version": "1.0", "ID": 9, "Message": "", "Pull": { "IndexCommand": 1, "PullCommand": "OpenDoor" } } |
| field | type | explain |  |
| Serial | String | serial number |  |
| ID | String | Custom device identification string |  |
| Now | String | The current system time of the device, represented by a string |  |
| TimeStamp | UInt32 | The current system time and timestamp of the device |  |
| Firmware | String | Firmware version |  |
| BuildTime | String | Firmware compilation time |  |
| PCBCode | String | Hardware code, used to distinguish hardware differences |  |
| Model | String | Equipment model name, functions of this product |  |
| ModelCode | UInt16 | Model code, see the model code table below |  |
| CardsInPacket | UInt8 | Maximum number of packages per batch transmission |  |
| SystemOption | UInt8 | Function code |  |
| DoorStatus | Array | The state of the door, for example, a double door controller array starts from the left of [0,0], where the first number is the first door and the second number is the second door. If there is no special explanation, the subsequent state class array data will be in this format. When both the door magnet and door relay are closed, it indicates that the door is closed. 0 open, 1 closed [0,0] indicates that both doors are open |  |
| Alarm Relay | Array | The status of the alarm relay, bit0: alarm relay; Bit1: Fire alarm relay. 0 off, 1 output |  |
| DoorRelay | Array | The status of the door relay, 0 closed, 1 output |  |
| Button | Array | Button input position status, 0 open, 1 closed |  |
| Sensor | Array | Door magnetic input position status, 0 open, 1 closed |  |
| LockOut | Array | Locked state, 1 locked, 0 unlocked |  |
| Input | UInt32 | Input interface status |  |
| InAlarm | UInt8 | Alarm input status, 0 open, 1 closed |  |
| InFire | UInt8 | Fire alarm input status, 0 open, 1 closed |  |
| InTamper | UInt8 | Anti disassembly input status, 0 open, 1 closed |  |
| InReset | UInt8 | Reset key status, 0 released, 1 pressed |  |
| OEM | UInt16 | Customer exclusive code |  |
| IndexCommand | UInt32 | Pull instruction number, generated by the server, returned by the controller's original value |  |
| CmdOK | UInt32 | Pull command execution result, 0 unsuccessful, 1 successful |  |
| ErrorCode | UInt32 | Error code for pulling instruction execution results. |  |
| Value | UInt32 | Pull command return parameter, used for batch transmission to return package number |  |

Software response with pull instruction data

| field | type | explain |
| --- | --- | --- |
| Pull | Json | Optional field, pull instruction content. If not, do not use this field as follows |
| IndexCommand | Uint32 | Pull the instruction number, and the controller returns the original value of the next heartbeat. |
| PullCommand | String | Pull command name |
|  |  | Specific pull instruction data, see each specific instruction, add corresponding field parameters |

# Controller sends events - swipe card records

Controller pushes record URLs

| Controller request |  |  | Software response |
| --- | --- | --- | --- |
| { "ID": 159, "Version": "1.0", "TaskNo": 10, "Method": "CardEvent", "Data":{ "Serial": "1G0095", "Time": "20221208104151", "DataType": 0, "Card": "9527", "EventType": 1, "Door": 1, "Reader": 0, "Exist": 0, Pass: 0, "APBInOut": 0, "APBOn": 0, Count: 0, Index: 1 } } |  |  | { "Method": "CardEvent", "Code": "0", "TaskNo": 10, "Version": "1.0", "ID": 159, "Message": "", "Data":{ Index: 1 } } |
| field | data type | explain |  |
| Serial | String | The serial number of the controller |  |
| Door | UInt8 | Door number, starting from 1 |  |
| Reader | UInt8 | This record was swiped on that reader, starting from 0 |  |
| Card | String | Card number for swiping cards |  |
| DataType | UInt8 | Data type for card swiping, 0 cards |  |
| Time | String | The time when the event occurred |  |
| EventType | UInt8 | Event type, as explained in the event type table |  |
| Index | UInt32 | The event record transmission flow must have the same response, and upon receiving the same return, the controller will delete this record |  |
| Count | UInt32 | The current number of records. |  |
| Exist | UInt8 | Does the card number in this record exist in the controller. |  |
| Pass | UInt8 | Is it allowed to pass this card swiping. |  |
| APBOn | UInt8 | Has anti backflow been enabled for this card swiping. |  |
| APBInOut | UInt8 | If anti backflow is enabled and effective, is this entry or exit. 0 in, 1 out. |  |

Controller sends events - alarm records

Controller pushes to record Url

| Controller request |  |  | Software response |
| --- | --- | --- | --- |
| { "Id": 1967, "Version": "1.0", "TaskNo": 10, "Method": "Alarm Event", "Data":{ "Serial": "1G0095", "Time": "20221208115704", "EventType": 59, "Door": 1, Count: 0, Index: 2 } } |  |  | { "Method": "Alarm Event", "Code": "0", "TaskNo": 10, "Version": "1.0", "Message": "", "Id": 1967, "Data":{ Index: 2 } } |
| field | data type | explain |  |
| Serial | String | The serial number of the controller |  |
| Door | UInt8 | Door number, starting from 1 |  |
| EventType | UInt8 | Event type, as described in the Event Type Table (Excel file) |  |
| Time | String | The time when the event occurred |  |
| Index | UInt32 | The event record transmission flow must have the same response, and upon receiving the same return, the controller will delete this record |  |
| Count | UInt32 | The current number of records. |  |

# Query controller status

The software actively queries the status of the controller

| Software response | Controller request |
| --- | --- |
| { "Method": "GetStatus", "TaskNo": 10, "Version": "1.0", "ID": 9, } | { "ID": 9, "Version": "1.0", "Method": "GetStatus", "TaskNo": 10, "Data":{ "Serial": "1G0095", "ID": "1C1110", DoorStatus: [0,0], "Input": 0, "Now": "2022-12-09 12:34:48", "Firmware": "0.1.121", "BuildTime": "Dec 9 2022 12:34:08", "PCBCode": "DD10", "Model": "G-ZJ1000TCP", TimeStamp: 1670589288, "CardsInPacket": 16, "SystemOption": 57, DoorRelay: [0,0], "Alarm Relay": [0,0], "Button": [0,0], "Sensor": [0,0], "Alarm In": 0, "FireIn": 0, "TamperIn": 0, Reset In: 0 } } |

The field explanation can be found in the heartbeat section.

# Command - Open and Close Doors

| Software instructions |  |  | Controller response |
| --- | --- | --- | --- |
| { "ID": 2017, "Data":{ "Door": 0, "Open": 1 }, "Version": "1.0", "TaskNo": 10, "Method": "OpenDoor" } |  |  | { "ID": 2017, "Code": 0, "TaskNo": 10, "Method": "OpenDoor", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data": {} } |
| field | data type | explain |  |
| Door | UInt8 | Door number, starting from 0 |  |
| Open | UInt8 | Action, 0 close the door, 1 open the door |  |

# Command - Long Open Door

| Software instructions |  |  | Controller response |
| --- | --- | --- | --- |
| { "Id": 2042, "Data":{ Door: 0 }, "Version": "1.0", "TaskNo": 10, "Method": "OpenDoorLong" } |  |  | { "ID": 2043, "Code": 0, "TaskNo": 10, "Method": "OpenDoorLong", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data": {} } |
| field | data type | explain |  |
| Door | UInt8 | Door number, starting from 0 |  |

# Command - Lock (Unlock) Door

| Software instructions |  |  | Controller response |
| --- | --- | --- | --- |
| { "ID": 2058, "Data":{ "Door": 0, "Lock": 1 }, "Version": "1.0", "Method": "LockDoor", "TaskNo": 10 } |  |  | { "ID": 2059, "Code": 0, "TaskNo": 10, "Method": "LockDoor", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data": {} } |
| field | data type | explain |  |
| Door | UInt8 | Door number, starting from 0 |  |
| Lock | UInt8 | Action, 0 off, 1 on |  |

# Instruction - Alarm Output

| Software instructions |  |  | Controller response |
| --- | --- | --- | --- |
| { "ID": 2114, "TaskNo": 10, "Data":{ "Open": 1 }, "Version": "1.0", "Method": "SetAlarm" } |  |  | { "ID": 2115, "TaskNo": 10, "Code": 0, "Method": "SetAlarm", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data": {} } |
| field | data type | explain |  |
| Open | UInt8 | Action, 0 off, 1 on |  |

# Command - Start (Close) Fire Alarm

| Software instructions |  |  | Controller response |
| --- | --- | --- | --- |
| { "ID": 2170, "TaskNo": 10, "Data":{ "Open": 1 }, "Version": "1.0", "Method": "SetFire" } |  |  | { "ID": 2171, "TaskNo": 10, "Code": 0, "Method": "SetFire", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data": {} } |
| field | data type | explain |  |
| Open | UInt8 | Action, 0 off, 1 on |  |

# Instruction - Direction Control (Card Reading Prohibited)

| Software instructions |  |  | Controller response |
| --- | --- | --- | --- |
| { "ID": 2170, "TaskNo": 10, "Data":{ "Door": 1, "Reader": 1, Keepout: 1 }, "Version": "1.0", "Method": "SetPass" } |  |  | { "ID": 2171, "TaskNo": 10, "Code": 0, "Method": "SetPass", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data": {} } |
| field | data type | explain |  |
| Door | UInt8 | Door number, starting from 0 |  |
| Reader | UInt8 | Control direction, 0 in, 1 out |  |
| Keepout | UInt8 | 1 is prohibited from passing, 0 is allowed to pass |  |

# Instruction - Sync Time

| Software instructions |  |  | Controller response |
| --- | --- | --- | --- |
| { "ID": 2208, "TaskNo": 10, "Data":{ "Second": 17, Day: 8, Minute: 5, Month: 12, "Hour": 12, Year: 2022 }, "Version": "1.0", "Method": "UpdateTime" } |  |  | { "ID": 2209, "TaskNo": 10, "Code": 0, "Method": "UpdateTime", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data": {} } |
| field | data type | explain |  |
| Second | UInt8 | Seconds, base 10 |  |
| Minute | UInt8 | Minute, decimal |  |
| Hour | UInt8 | Time, base 10 |  |
| Day | UInt8 | Day, decimal |  |
| Month | UInt8 | Month, decimal |  |
| Year | UInt16 | Year, decimal |  |

# Command - Controller Restart

| Software instructions | Controller response |
| --- | --- |
| { "ID": 2222, "TaskNo": 10, "Data": {}, "Version": "1.0", "Method": "Restart" } | { "ID": 2223, "TaskNo": 10, "Code": 0, "Method": "Restart", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data": {} } |

# Instruction - Controller initialization

| Server Instructions | Controller response |
| --- | --- |
| { "ID": 13, "TaskNo": 10, "Data": {}, "Version": "1.0", "Method": "Reset" } | { "ID": 14, "TaskNo": 10, "Code": 0, "Method": "Reset", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data": {} } |

# Instruction - Send data through 485

| Software instructions |  |  | Controller response |
| --- | --- | --- | --- |
| { "ID": 18, "TaskNo": 10, "Data":{ "Data": "MTIzNDU2QUJD" }, "Version": "1.0", "Method": "TransmitTo485" } |  |  | { "ID": 19, "TaskNo": 10, "Code": 0, "Method": "TransmitTo485", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data": {} } |
| field | data type | explain |  |
| Data | String | The BASE64 encoded string of the data to be sent through 485 |  |

Instruction - Broadcast Voice

| Software instructions |  |  | Controller response |
| --- | --- | --- | --- |
| { "ID": 18, "TaskNo": 10, "Data":{ "Data": "abc" }, "Version": "1.0", "Method": "Speak" } |  |  | { "ID": 19, "TaskNo": 10, "Code": 0, "Method": "Speak", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data": {} } |
| field | data type | explain |  |
| Encoding | Uint8 | Encoding of broadcast text, 0=GB2312, 4=UTF8 |  |
| Ring | Uint8 | Ring before broadcasting, 0 does not ring, none defaults to 1 |  |
| Data | String | Broadcast text string, with a maximum length of 128 bytes |  |

# Instruction - Delete All Cards

| Software instructions | Controller response |
| --- | --- |
| { "ID": 32, "TaskNo": 10, "Data": {}, "Version": "1.0", "Method": "ClearCards" } | { "ID": 33, "TaskNo": 10, "Code": 0, "Method": "ClearCards", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data": {} } |

This command requires different execution times depending on the model, ranging from 0.2 seconds to 15 seconds, but will receive an immediate response after sending the command. It is recommended that the software wait for the next heartbeat before issuing other commands.

# Instruction - Delete 1 Card

| Software instructions |  |  | Controller response |
| --- | --- | --- | --- |
| { "ID": 64, "TaskNo": 10, "Data":{ Card: 444855280, CardIndex: 0 }, "Version": "1.0", "Method": "DeleteCard" } |  |  | { "ID": 65, "TaskNo": 10, "Code": 0, "Method": "DeleteCard", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data":{ CardIndex: 0 } } |
| field | data type | explain |  |
| Card | UInt32 | Card number to be deleted, decimal number |  |
| CardIndex | UInt32 | Optional parameter, delete data with specified number. If this field exists, then Card is not required. This parameter takes priority and is recommended |  |

Attention:

There are two methods and modes for card management: using card numbers and not using card numbers. This is determined by the management software. If a card number is used, the card number will be used for deletion, addition, and batch addition. Otherwise, the card number will not be used. Using card numbering can improve performance, especially when there are a large number of cards. It is strongly recommended to use card numbering mode, but some software architectures may not be suitable for using card numbering. The card number in this agreement uses the field name "CardIndex".

Definition: Card number

In computer software, it is necessary to maintain a list of people, where each record represents a person's information and generates a unique ID, which is the card number. The card number must start from 0 and must not be interrupted, repeated, or exceed the card capacity of the controller (approximately 30000 to 60000). So when the software deletes a card, it only leaves the number blank. When adding a card, you need to first use the previously deleted number.

# Instruction - Add 1 Card

| Software instructions |  |  | Controller response |
| --- | --- | --- | --- |
| { "ID": 44, "TaskNo": 10, "Data":{ "CardIndex": 0, "TZ1": 1, "TZ2": 0, Card: 444855280, "Password": "87654321", "BeginTime": "3007232359", "EndTime": "2212161647", "Name": "Card A", "Status": 1 }, "Version": "1.0", "Method": "AddCard" } |  |  | { "ID": 45, "TaskNo": 10, "Code": 0, "Method": "AddCard", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data":{ CardIndex: 0 } } |
| field | data type | explain |  |
| CardIndex | UInt32 | Optional parameter, starting from 0 and not exceeding the controller capacity of -1. If there is one, save it according to the specified number. Otherwise, search for the card number to overwrite it. If it does not exist, automatically search for a blank space to save it. |  |
| Card | UInt32 | Card number, decimal of a 4-byte integer |  |
| TZ1 | UInt8/UInt16 | Open time - The first gate of the 124th gate controller, expressed in binary bits, Each bit represents 1 open time, corresponding to the entire open schedule in order, 0 not selected, no permission 1 selected |  |
| TZ2 | UInt8/UInt16 | Open hours -24th door controller 2nd door |  |
| TZ3 | UInt8 | Open time selection -4th door controller 3rd door |  |
| TZ4 | UInt8 | Open Time Selection - The 4th door of the 4-door controller has 8 bits, each representing 1 open time, corresponding to the entire open schedule in order. |  |
| Password | String | Optional parameter, password does not exceed 8 digits |  |
| BeginTime | String | Optional parameter, valid from yyMMddHHmm; 10 base string; In byte order, they are respectively year, month, day, hour, and hour. For example, 3001012359:30 represents 2030, 01 represents January, 01 represents the 1st day, 23 represents 23:00, 59 represents 59 minutes |  |
| EndTime | String | Optional parameter, valid until yyMMddHHmm; A decimal string, year needs to be subtracted by 2000 first. |  |
| Name | String | Optional parameter, personnel name |  |
| Status | UInt8 | Optional parameter, personnel activation status 1 available, 0 unavailable |  |

Return results

| CardIndex | UInt32 | Return parameter, the actual saved card number, which is the position of the card |
| --- | --- | --- |

Optimization Tip: If you need to download all or the vast majority of cards, you can delete all cards first before downloading, which can improve performance by at least 1 time or more. Clear all before adding, for higher performance. It is also suitable for batch addition.

# Command - Update card status, anti backflow function

| Software instructions |  |  | Controller response |
| --- | --- | --- | --- |
| { "ID": 44, "TaskNo": 10, "Data":{ "CardIndex": 0, Card: 444855280, "APBInOut": 0, "APBOn": 0, "Times": [1,2,3,4,5], "Time": "1216164735" }, "Version": "1.0", "Method": "UpdateCardAPB" } |  |  | { "ID": 45, "TaskNo": 10, "Code": 0, "Method": "UpdateCardAPB", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data":{ CardIndex: 0 } } |
| field | type | explain |  |
| CardIndex | UInt32 | Optional parameter, starting from 0 and not exceeding the controller capacity of -1. If available, save according to the specified number; otherwise, automatically search for a blank space to save. |  |
| APBOn | UInt8 | Is anti backflow enabled for this card? 0 is not enabled, 1 is enabled |  |
| APBInOut | UInt8 | Set whether the card is in or out. 0 in, 1 out. |  |
| Times | Array | Optional parameter, based on the number of card swipes during the first 5 open hours |  |
| Time | String | Optional parameter, time of entry and exit MMddHHmmss; A decimal string without a year. |  |

# Command - Search for 1 card

| Software instruction |  |  | Controller response |
| --- | --- | --- | --- |
| { "ID": 82, "TaskNo": 10, "Data":{ Card: 444855280 }, "Version": "1.0", "Method": "SearchCard" } |  |  | { "ID": 83, "TaskNo": 10, "Code": 0, "Method": "SearchCard", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data":{ "CardIndex": 0, Card: 444855280, "Status": 1, "TZ1": 1, "BeginTime": "3007232359", "EndTime": "3001012359", "Password": "87654321", "Name": "Card A" } } |
| field | data type | explain |  |
| Card | UInt32 | Optional parameters, searched card number, decimal number |  |
| Password | String | Optional parameters, search password |  |
| CardIndex | UInt32 | Optional parameters, card number, i.e. card position |  |

Choose one of three parameters!

Return results

| Card | UInt32 | Card number, decimal number |
| --- | --- | --- |
| CardIndex | UInt32 | Card number, which refers to the position of the card |
| Status | UInt8 | Search Results - Status |
| TZ1 | UInt8/UInt16 | Open time selection -124th door controller, first door, see instructions for adding cards |
| TZ2 | UInt8/UInt16 | Open time selection -24th door controller, 2nd door |
| TZ3 | UInt8 | Open time selection -4th door controller 3rd door |
| TZ4 | UInt8 | Open time selection -4th door controller 4th door |
| BeginTime | String | Validity period starts yyMMddHHmm; 10 base string; In byte order, they are year, month, day, and hour, for example, 3001012359: 30 represents 2030, 01 represents January, 01 represents the 1st day, 23 represents 23:00, 59 represents 59 minutes |
| EndTime | String | The expiration date is yyMMddHHmm; A decimal string, year needs to be subtracted by 2000 first. |
| Password | String | password |
| Name | String | name |

# Instruction - Batch Card Addition (with numbering mode, recommended)

| Software instruction |  |  | Controller response |
| --- | --- | --- | --- |
| { "ID": 106, "TaskNo": 10, "Data": { CardofPack: 16, "PackIndex": 0, "TZ1": [1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1], "TZ2": [1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1], Card:[ 2001, 2002, 2003, 2004, 2005, 2006, 2007, 2008, 2009, 20102011201122013201420152016, "Password":[ "5000", "5001", "5002", "5003", "5004", "5005", "5006", "5007", "5008", "5009", "5010", "5011", "5012", "5013", "5014", "5015"], "BeginTime": ["3001012359", "3001012359", "3001012359", "3001012359"], "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359"], "EndTime": ["3001012359", "3001012359", "3001012359", "3001012359"], "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359"], "Name": [Card A1, Card A2, Card A3, Card A4], "Card A5", "Card A6", "Card A7", "Card A8", "Card A9", "Card A10", "Card A11", "Card A12", "Card A13", "Card A14", "Card A15", "Card A16"] }, "Version": "1.0", "Method": "AddAllCards" } |  |  | { "ID": 107, "TaskNo": 10, "Code": 0, "Method": "AddAllCards", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data":{ PackIndex: 0 } } |
| field | data type | explain |  |
| CardofPack | Uint8 | The number of cards in the current package should not exceed the CardsInPacket value in the heartbeat. |  |
| PackIndex | Uint32 | Batch transmission, batch number, starting from 0, the controller's original value response indicates successful transmission. |  |
| Card | Array | Uint32 integer array, decimal card number |  |
| TZ1 | Array | Uint8/Uint16 integer array, Permission opening time selection -124th door controller, first door, see instructions for adding cards |  |
| TZ2 | Array | Uint8/Uint16 integer array, permission opening time selection -24 gates, controller 2nd gate |  |
| TZ3 | Array | Uint8 integer array, permission opening time selection -4th gate controller 3rd gate |  |
| TZ4 | Array | Uint8 integer array, permission opening time selection -4th gate controller 4th gate |  |
| Password | Array | Optional parameters, string array, each person's password |  |
| Name | Array | Optional parameters, string array, each person's name |  |
| BeginTime | Array | Optional parameter, string array, decimal string starting from validity period; In byte order, they are respectively year, month, day, hour, and hour. For example, 3001012359:30 represents 2030, 01 represents January, 01 represents the 1st day, 23 represents 23:00, 59 represents 59 minutes |  |
| EndTime | Array | Optional parameter, valid until yyMMddHHmm; A decimal string, year needs to be subtracted by 2000 first. |  |

In this mode, the performance is highest, and the number of each packet must be CardsInPacket, except for the last packet. The data is saved consecutively in numerical order. The number of the first person in each package is PackIndex * CardsInPacket, and the number of subsequent personnel in the package is increased by 1. This mode is suitable for managing personnel by number, where each person has a corresponding fixed number and is continuous. Alternatively, it can be used to download all personnel.

# Instruction - Batch add personnel (no numbering mode, not recommended)

| Software instructions |  |  | Controller response |
| --- | --- | --- | --- |
| { "ID": 106, "TaskNo": 10, "Data": { CardofPack: 16, "PackIndex": 0, "TZ1": [1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1], "TZ2": [1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1,1], Card:[ 2001, 2002, 2003, 2004, 2005, 2006, 2007, 2008, 2009, 20102011201122013201420152016, "Password":[ "5000", "5001", "5002", "5003", "5004", "5005", "5006", "5007", "5008", "5009", "5010", "5011", "5012", "5013", "5014", "5015"], "BeginTime": ["3001012359", "3001012359", "3001012359", "3001012359"], "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359"], "EndTime": ["3001012359", "3001012359", "3001012359", "3001012359"], "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359", "3001012359"], "Name": [Card A1, Card A2, Card A3, Card A4], "Card A5", "Card A6", "Card A7", "Card A8", "Card A9", "Card A10", "Card A11", "Card A12", "Card A13", "Card A14", "Card A15", "Card A16"] }, "Version": "1.0", "Method": "AddListCards" } |  |  | { "ID": 107, "TaskNo": 10, "Code": 0, "Method": "AddListCards", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data":{ PackIndex: 0 } } |
| field | data type | explain |  |
| CardofPack | Uint8 | The number of cards in the current package should not exceed the CardsInPacket value in the heartbeat. |  |
| PackIndex | Uint32 | Batch transmission, batch number, starting from 0, the controller's original value response indicates successful transmission. |  |
| Card | Array | Uint32 integer array, decimal card number |  |
| TZ1 | Array | Uint8/Uint16 integer array, permission opening time selection -124th gate controller 1st gate, See instructions for adding cards |  |
| TZ2 | Array | Uint8/Uint16 integer array, permission opening time selection -24 gates, controller 2nd gate |  |
| TZ3 | Array | Uint8 integer array, permission opening time selection -4th gate controller 3rd gate |  |
| TZ4 | Array | Uint8 integer array, permission opening time selection -4th gate controller 4th gate |  |
| Password | Array | Optional parameters, string array, each person's password |  |
| Name | Array | Optional parameters, string array, each person's name |  |
| BeginTime | Array | Optional parameter, string array, decimal string starting from validity period; In byte order, they are respectively year, month, day, hour, and hour. For example, 3001012359:30 represents 2030, 01 represents January, 01 represents the 1st day, 23 represents 23:00, 59 represents 59 minutes |  |
| EndTime | Array | Optional parameter, valid until yyMMddHHmm; A decimal string, year needs to be subtracted by 2000 first. |  |

The number of elements in the array must be consistent. This mode is suitable for management without a number. Personnel search by card number as the unique key value. If it exists, it will be overwritten, otherwise it will be saved to a blank space. Each instruction takes 3-5 seconds to execute, and the performance improvement in this mode is limited. It is not recommended to use.

# Instruction - Initialize All Open Hours

| Software instructions |  |  | Controller response |
| --- | --- | --- | --- |
| { "ID": 256, "TaskNo": 10, "Data":{ Door: 0 }, "Version": "1.0", "Method": "DeleteTimeZone" } |  |  | { "ID": 257, "TaskNo": 10, "Code": 0, "Method": "DeleteTimeZone", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data": {} } |
| field | data type | explain |  |
| Door | UInt8 | Door number, starting from 0 |  |

# Instruction - Add Open Time

| Software instructions |  |  | Controller response |
| --- | --- | --- | --- |
| { "ID": 274, "TaskNo": 10, "Data":{ "Door": 0, "Index": 0, "Identify": 0, "Group": 0, "FrmHour": 0, "FrmMinute": 0, "ToMinute": 59, "ToHour": 23, "Passback": 0, "Holiday": 1, Year: 30, "Month": 1, Day: 1, "Week": [1,1,1,1,1,1,1,1] }, "Version": "1.0", "Method": "AddTimezone" } |  |  | { "ID": 275, "TaskNo": 10, "Code": 0, "Method": "AddTimezone", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data": {} } |
| field | data type | explain |  |
| Door | UInt8 | The serial number of the door, starting from 0 |  |
| Index | UInt8 | Number of opening hours, starting from 0 |  |
| Identify | UInt8 | Identification method, select only one 1: Single card recognition 2: Kaga password 3: Timed door opening and closing 7: Timed alarm 8: Password 9: Card or password 17: Button time |  |
| Group | UInt16 | Group number |  |
| FrmHour | UInt8 | At the start time, decimal |  |
| FrmMinute | UInt8 | Start time minute, decimal |  |
| ToHour | UInt8 | At the end of the time, decimal |  |
| ToMinute | UInt8 | End time minute, decimal |  |
| Year | UInt16 | End date year of open hours, decimal |  |
| Month | UInt8 | Month, Decimal |  |
| Day | UInt8 | Day, Decimal |  |
| Holiday | UInt8 | Is holiday enabled? 1 Yes 0 No |  |
| Week | Array | Select an array for the week, in the order of Sunday, Monday, Tuesday... Saturday; [1,1,1,1,1,1,1,1] 1 is enabled and 0 is not enabled |  |
| Passback | UInt8 | Anti submarine return 1 enabled 0 not enabled |  |

# Instruction - Update Gate Parameters

| Software instructions |  |  | Controller response |
| --- | --- | --- | --- |
| { "ID": 286, "TaskNo": 10, "Data":{ "MCardsInOut": 0, "OutTime": 5, "Alarm Mask": [1,1,1,1], "MCards": 0, "Door": 0, "OpenTime": 1, "Alarm Time": 10, "ToolongAlarm": 0 }, "Version": "1.0", "Method": "SetDoor" } |  |  | { "ID": 287, "TaskNo": 10, "Code": 0, "Method": "SetDoor", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data": {} } |
| field | data type | explain |  |
| Door | UInt8 | Door number, starting from 0 |  |
| OpenTime | UInt16 | Opening time |  |
| OutTime | UInt8 | Time range for detecting door timeout after closing the door |  |
| Alarm Mask | Array | Alarm type generates alarm output (alarm relay action), with 4 options: 1 enabled, 0 not enabled; Order: [Door alarm, alarm for too long opening time, alarm for invalid card, alarm for invalid time] |  |
| Alarm Time | UInt16 | Alarm output alarm time |  |
| ToolongAlarm | UInt8 | Is the function of opening the door for too long enabled? 1 enabled, 0 not enabled |  |
| MCards | UInt8 | Multi card door opening, greater than 1 indicates activation |  |
| MCardsInOut | UInt8 | When enabling multi card door opening, the enabled directions are 1 in direction, 2 out direction, and 3 in and out directions |  |

# Software sends instructions - update controller parameters

| Software instructions |  |  | Controller response |
| --- | --- | --- | --- |
| { "ID": 298, "TaskNo": 10, "Data":{ "FireTime": 0, "LockEach": 0, "Duress": "5874", "Alarm Time": 10 }, "Version": "1.0", "Method": "SetController" } |  |  | { "ID": 299, "TaskNo": 10, "Code": 0, "Method": "SetController", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data": {} } |
| field | data type | explain |  |
| LockEach | UInt8 | 0: No interlock, this field is not available for single door and gate controllers; 1: 1-2 door interlock 2: 3-4 door interlock 3: 1-2-3 door interlock 4: 1-2-3-4 door interlock |  |
| FireTime | UInt16 | After the fire alarm is triggered, the output time length of the alarm relay |  |
| Duress | String | Seize password, numeric string |  |
| Alarm Time | UInt16 | The duration of the alarm relay output after the alarm input is triggered |  |
| Title | String | The title displayed on the LCD interface is only available for all-in-one machines |  |

# Software Send Command - Update Controller Network Configuration

| Software instructions |  |  | Controller response |
| --- | --- | --- | --- |
| { "ID": 298, "TaskNo": 10, "Data":{ Protocol: 1, "ServerPort": 89, "Server": "w.menjin. net", "UrlHeart": "/data/Acs. aspx? Method=GetStatus&", "UrlRecord": "/data/Acs. aspx? Method=SearchCardAcs" }, "Version": "1.0", "Method": "SetNetwork" } |  |  | { "ID": 299, "TaskNo": 10, "Code": 0, "Method": "SetNetwork", "Message": "", "Serial": "1G0095", "Version": "1.0", "Data": {} } |
| field | data type | explain |  |
| Protocol | UInt8 | The communication protocol used by the controller. 0x01: HTTP 0x20: TCP 0x40: MQTT |  |
| ServerPort | UInt16 | Report the port of the server |  |
| Server | String | Report the address, IP, or domain name of the server without the HTTP://prefix. |  |
| UrlHeart | String | Report server heartbeat status address |  |
| UrlRecord | String | Report server record address |  |

This is the end of the file!